RFID Reader API Technical Reference Manual

V1.4

2025.03.15

Revision record

| Date | Revision | Description | Author |
| --- | --- | --- | --- |
| 2024/10 | V1.1 | - Document organization, publishing revised versions |  |
| 2024/12 | V1.2 | - 1. Add IMSI field to Mg4GIMEIConfig - 2. Add MongoDB Port DwellMaxTimeConfig MsgTemperatureQuery， Applicable to X model module equipment |  |
| 2025/01 | V1.3 | - 1. Add new MgNetProtocolType MsgMQTTAuthorizeParam、MsgMQTTBaseParam， Configuration of communication protocols in network port connection mode |  |
| 2025/03 | V1.4 | - Adjust the communication encoding method (MgCommunicationModeConfig) - Added reading mode "EX10 mode", which only takes effect on modules ending with "X" in the model - Adjust the frequency band and frequency. - Add wireless connection method selection (FsWireless CommunicationModeConfig) - Delete 3.6.9 and 3.6.10 APIs and use 3.6.3 and 3.6.4 uniformly |  |

catalogue

1、 Overview	5

1.1 Definition of Abbreviations	5

1.2 Scope of Application	5

1.3 Reading instructions	5

1.4 Description of API versions	5

1.5 Development Environment Description	5

2、 Public Call Instructions	6

2.1 Overview of API Usage	6

2.1.1 Reader operation class	6

2.1.2 API Message Base Classes (AbstractHostMessage, AbastractReaderMessage)	6

2.1.3 API namespace description	6

2.1.4 API Process Diagram	7

2.2 Reader operation class	10

2.2.1 Explanation of Class Construction Methods	10

2.2.2 Common Attribute Description for Class	11

2.2.3 Common Method Description	13

2.2.4 General Event Description	14

2.3 Message Instruction Base Class (AbstractHostMessage)	17

2.4 Configuration file (DeviceCfg. xml)	19

3、 Protocol API (Instruction Message) List	20

3.1 Device Connection Parameter Query and Setting	20

3.1.1 Set/Query RS232 BaudRateConfig Class	20

3.1.2 Setting/Querying BaudRateConfig Class - For Some Old Devices	21

3.1.3 Setting/Querying RS485 BaudRateConfig Class	21

3.1.4 Setting/querying the RS-485 address of the local machine (using the sgRs485AutConfig class)	22

3.1.5 Setting/Querying the IP Address of the Network Port (in the MgIpAddressConfig class)	22

3.1.6 Setting/Querying Network Port Server/Client Mode (using the MgTcpModeConfig class)	23

3.1.7 Setting/Querying whether DHCP is enabled on the network port (using the HttpDHCPModeConfig class)	23

3.1.8 Querying the MAC address of the network port (using the MonsMacConfig class)	24

3.1.9 Network port parameter reset command (MsDeviceResetByUdp class)	24

3.1.10 Setting/querying the WIFI server/client mode on the application side (using the MongoDB WiiModeConfig class)	25

3.1.11 Setting/Querying the WIFI IP Address of the Application Side (using the MsWifiIpAddressConfig class)	25

3.1.12 Setting/Querying Application Side WIFI Hotspot Parameters (MsWifiHotspotConfig Class)	26

3.1.13 Setting/querying the status of the WIFI network card on the application side (using the MongoDB WifiStatusConfig class)	27

3.1.14 Querying the MAC address of the WIFI on the application side (using the MsWifiMacConfig class)	27

3.1.15 Setting/Querying 4G Server Domain/IP Address (using the MsServerHostConfig class)	28

3.1.16 Setting/Querying 4G Access Point APN (MgApnConfig class)	28

3.1.17 Querying 4G IMEI (using the sg4GIMEIMEIConfig class)	28

3.1.18 Setting/Querying the Communication Protocol Type under the Network Port (e.g., MsNetProtocolType class)	29

3.1.19 Setting/querying MQTT connection authentication parameters (using the sgMQTTAuthorizeParam class)	29

3.1.20 Setting/Querying MQTT Basic Parameters (MgMQTTBaseParam Class)	30

3.1.21 Setting/Querying Wireless Connection Method (FsWireless CommunicationModeConfig)	30

3.2 Equipment parameter query and setting	31

3.2.1 Query module software and hardware versions (MgReaderVersions Query class)	31

3.2.2 Querying the software and hardware versions of the application processor (using the MgCpuVersions Query class)	31

3.2.3 Querying the serial number of the read-write module product (using the sgReaderModulus SNQuery class)	32

3.2.4 Querying the serial number of the reader/writer product (using the sgReaderSNQuery class)	32

3.2.5 Querying RFID working status (using the MonsRfidStatus Query class)	32

3.2.6 Setting/querying the working status of the buzzer (using the MgReaderBeepConfig class)	34

3.2.7 Setting/querying the working mode of the reader/writer (using the SchoolReaderWorkModeConfig class)	34

3.2.8 Setting/Querying Reader Tag Reading Mode (MgScanModeConfig Class)	35

3.2.9 Setting/Querying Sleep State (using the MgSleepStateConfig class)	36

3.2.10 Setting/querying signal strength (RSSI) filtering threshold (using the sgRssiThresholdConfig class)	36

3.2.11 Setting/Querying Duplicate Tag Filtering Time (using the MgFilteringTimeConfig class)	36

3.2.12 Setting/Querying Interval Reading Label Time (in the MgIntervalTimeConfig class)	37

3.2.13 Setting/Querying Idle Waiting Time (using the MgIdleTimeConfig class)	38

3.2.14 Setting/Querying Heartbeat Command Configuration (MgKeepAliveConfig Class)	38

3.2.15 Setting/Querying Staged Tag Reading Configuration Parameters (MgReadTagCfgParamConfig Class)	39

3.2.16 Setting/Querying the working mode of the card issuer (using the MgTagInitConfig class)	39

3.2.17 Setting/Querying UTC Time (in the MgUtoConfig class)	40

3.2.18 Reader/writer restart (using the MgAppCpuReset class)	41

3.2.19 Restore Factory Settings (using the sgResetToFactoryDefault class)	41

3.2.20 Setting/querying the maximum dwell time of the antenna (in the category of MgPortDwellMaxTimeConfig)	41

3.2.21 Querying device temperature (using the FsTemperatureQuery class)	41

3.3 Equipment frequency band&antenna parameter query and setting	42

3.3.1 Querying the number of antennas and power threshold of the reader/writer (using the MonsReaderCapability Query class)	42

3.3.2 Setting/Querying Operating Frequency Bands (using the MonsUhfBandConfig class)	42

3.3.3 Setting/Querying the Antennas Used (in the MgAntennaConfig class)	43

3.3.4 Querying the connection status of antenna ports (using the MonsAntennaStatusQuery class)	44

3.3.5 Setting/Querying Operating Frequency (in the MgFrequencyConfig class)	44

3.3.6 Setting/Querying Port Power Values (in the MonsPowerConfig class)	46

3.3.7 Setting/Querying Port Residency Time (using the sgPortDwellTimeConfig class)	47

3.3.8 Setting/querying the power value for reading labels on a certain port (using the MonsReadPowerConfig class)	47

3.3.9 Setting/querying the power value of antenna port label writing (in the WriteWritePowerConfig class)	48

3.4 Equipment Gen 2 parameter query and setting	49

3.4.1 Setting/Querying Supported Air Interface Protocols (MgAirProtocolConfig Class)	49

3.4.2 A Setting/Querying Communication Protocol Encoding Mode (MgCommunicationModeConfig Class)	49

3.4.3 Setting/querying communication WiegandParameterConfig class	50

3.4.4 Setting/querying the starting Q value (using the sgQValueConfig class)	50

3.4.5 Setting/Querying the Session of a Port (using the MsSessionConfig class)	51

3.4.6 Setting/Querying TagFocus Options (MgTagFocusConfig Class)	51

3.4.7 Setting/Querying FastID Options (using the MonsFastIDConfig class)	52

3.5 GPIO parameter query and setting	52

3.5.1 Querying GPIO Interface Definitions (MgGpioDefinitionQuery Class)	52

3.5.2 Querying GPIO Input Level (in the MgGpiQuery class)	53

3.5.3 Setting GPIO output levels (for the MgGpoConfig class)	53

3.5.4 Setting/Querying GPIO Input Triggered Label Operations (MgGpiTriggerConfig Class)	54

3.6 Label Operation	55

3.6.1 Tag Stop Reading (MonsPowerOff Class)	55

3.6.2 Setting/Querying 6C Tag Reading Contains Fields (Mg6CTagFieldConfig Class)	55

3.6.3 6C Tag Inventory (MgTagInventory class)	56

3.6.4 6C Tag Universal Reading (MgTagRead class)	57

3.6.5 6C Tag Selection (MgTagSelect class)	58

3.6.6 6C Tag Writing (MgTagWrite class)	59

3.6.7 6C Tag Lock (MgTagLock class)	59

3.6.8 6C Tag Destruction (MgTagKill Class)	60

3.6.9 6C Tag and PHASE Value Reading (MgTagReadWithPhase Class)	61

3.6.10 6C Tag Inventory EPC Area and PHASE Value (MgTagInventoryWithPhase Class)	61

4、 Common application functions and business scenario examples	62

4.1 Reader connection	62

4.2 Label Reading	63

4.3 Other label operations (write, lock, destroy)	65

4.3.1 Label Writing	65

4.3.2 Label Lock/Unlock	66

4.3.3 Label Destruction (Handle with Caution)	67

4.4 GPIO Applications	67

4.4.1 GPI	67

4.4.2 GPO	68

4.5 Reading Mode Description	68

4.6 Other Common Configuration Instructions	69

4.6.1 RSSI	69

4.6.2 Label illumination	69

4.6.3 Other common functions (tag deduplication, interval reading)	70

Appendix A: Example of Label Reading and Writing Process Controlled by Upper Computer Reader/Writer	71

Appendix B: Error Code Description	71

Appendix C: List of API Functions for Each Version	72

Appendix D: List of Partial Label Chip Manufacturers	74

- overview

  - Definition of Abbreviations

VRP：UHF Reader Protocol， Reader Data Transmission Control Protocol.

API： Application Programming Interface. Can be used for secondary development, it encapsulates the specific communication protocol of the reader/writer. Unless otherwise specified, the messages, instructions, and messages mentioned in this article are equivalent to APIs.

Upper computer: refers to a personal computer [PC] or other control terminal used for data exchange with a reader/writer, that is, client software.

Lower computer: refers to a reader/writer.

COM（cluster communication port）： Serial communication port.

TCP（Transfer Control Protocol）： Transmission Control Protocol.

RSSI（Received Signal Strength Indication）： Label echo intensity.

UTC(UTC, Universal Time Coordinated)： During general coordination. The default starting point is 1970-1-1 00:00:00.

Word: Two bytes make one word.

  - scope of application

The reader/writer models applicable to this document are:

  - Read instructions

The code description section of this reference manual is mainly in C # language without special instructions. For other languages, refer to the C # language description. The usage of the Android version API is the same as that of the JAVA version API.

  - API Version Description

Provide three programming language APIs to choose from

C#： Contains API.dll and Core.dll files, based on Microsoft NET Framework v4.0 framework development.

Java: UhfRfidAPI.jar file, written based on jdk-1.8.

Android: rfid.uhfapi.jar file, minimum SDK version 16, supports Android 4.1 and above platforms.

Android y2005: rfid.uhfapi.Y2005. jar file, minimum SDK version 16, supports Android 4.1 and above platforms.

C++: See separate document

  - Development Environment Description

Provide three programming language APIs to choose from

C #: UhfRfidTest project file, written based on Visual Studio 2019.

Java: UhfRfidTest project file, written based on Eclipse IDE for Eclipse Commitors Version: 4.11.0.

Android: RfidUhfTest project file, written based on Android Studio 3.5.

Android y2005: RfidUhfTestY2005 project file, written based on Android Studio 3.5

C++: See separate document

- Public Call Instructions

  - Overview of API Usage

This API adopts object-oriented design and supports calling multiple readers/writers. Each reader/writer is instantiated with a Reader class control, and each Reader example does not interfere with each other. API mainly serves as the interface between client software and reader/writer communication, including reader/writer classes and a series of message classes. Through these classes, functions such as establishing, disconnecting, sending, and receiving messages with the reader/writer are implemented.

This API is divided into reader/writer operation class and communication message class as a whole.

    - Reader operation class

The instructions for use are as follows

- Specify the reader/writer connection and construct the corresponding reader/writer operation: public Reader(String readerName, IPort commPort)

- Establish a reader/writer connection: public ConnectResponse Connect()

- Registration tag data reception event: public event InventoryReceivedHandle OnInventoryReceived

- Send various command messages: public bool Send(IHostMessage msg)

- Disconnect the reader/writer connection: public void Disconnect()

Please refer to the detailed introduction2.2 Reader operation class”

    - API Message Base Classes (AbstractHostMessage, AbastractReaderMessage)

- The communication between the upper computer and the reader/writer is based on message class implementation, that is, communication is completed by sending and receiving through the Reader. A brief explanation is as follows

- Upper computer message class ->AbstractHostMessage ->IHostMessage ->IReaderMessage. The vast majority of APIs in this document are for upper computer messaging, please refer to for detailed introduction2.3 Message Instruction Base Class (AbstractHostMessage)”

- The reader/writer actively uploads message classes ->AbastractReaderMessage ->IReaderMessage. Please refer to the previous article and do not provide a separate explanation.

- The upper computer message class, as a parameter of the Send method of the Reader class, completes instruction interaction

- Suggest creating a new message class for each message. If it needs to be reused, the IReadeMaessage interface provides a Clone method to initialize the message.

    - API namespace description

Unless otherwise specified, follow the following rules

- Reader class C #: located in NetAPI In the Protocol.VRP namespace

JAVA： Located in JavaAPI In the Protocol.VRP package

Android： Located in the rfid.uhfapi-protocol.vrp package

Android y2005： Located in the package rfid.uhfapi.Y2005. protocol. vrp

- All subclasses of IHostMessage, C # located in NetAPI In the Protocol.VRP namespace

JAVA： Located in JavaAPI In the Protocol.VRP package

Android： Located in the rfid.uhfapi-protocol.vrp package

Android y2005： Located in the package rfid.uhfapi.Y2005. protocol. vrp

- All parameters and enumeration classes, C # is located in NetAPI In the Entities namespace

JAVA： Located in JavaAPI In the Entities package

Android： Located in the rfid.uhfapi.entities package

Android y2005： Located at rfid.uhfapi.Y2005 In the entities package

- When using the APIs of Android and Android y2005 versions, Context needs to be passed in, and how to pass it in

new ApiApplication().init(getApplicationContext());

    - API Process Diagram

- Overall process of API usage

As shown in the picture:

In the above figure, there are two ways for the reader/writer to send instructions after establishing a connection:

- One method is to synchronize and wait after sending a message, with a specified waiting time of 1 second by default

- One approach is to return control to the system without waiting for the reader to execute the message.

In addition, for messages actively uploaded by the reader/writer, the event callback mechanism can also be used to asynchronously receive reader/writer information; This type of information includes tag scan data, IO trigger data, etc.

- Synchronize message flow in API

As shown in the picture:

In the above figure, the process of sending instructions is as follows:

- Instantiate the message class, which inherits from the IHostMessage interface and passes message parameters using the constructor parameters of the concrete class;

- Use the Send method in the Reader class to send instructions to the reader/writer;

- Waiting for message return (this step is automatically completed by the API without customer intervention)

- The message is returned to the message class instantiated in step 1), and the content of the returned message is obtained using the ReceptdMessage property (JAVA: getReceptdMessage method) of this class;

- If the message execution fails or times out, the Send method returns False and returns an error message through the Error Info property (JAVA: getError Info method) of the IHostMessage interface

- Process of sending message without waiting for return in API

As shown in the picture:

In the above figure, the process of sending instructions is as follows:

- Instantiate a message class that inherits from the IHostMessage interface, with the IsReturn variable value set to false (JAVA: setIsReturn method set to false). Use the constructor parameters of the specific class to pass the message parameters;

- Use the Send method in the Reader class to send instructions to the reader/writer;

- After sending the instruction, the method directly returns whether the instruction was sent successfully without waiting for a response from the reader/writer

- If successful, the Send method returns True and the program continues to execute

- If it fails, the Send method returns False and returns an error message through the Error Info property (JAVA: getError Info method) of the IHostMessage interface; The program continues to execute downwards

- The process of receiving active information uploads from readers/writers in the API

As shown in the picture:

In the above figure, the process of receiving the active message upload from the reader/writer is as follows:

- After the reader/writer successfully establishes a connection, add a listening event (OnInventoryReceived);

- When tag information is uploaded to the reader/writer, this event will be triggered;

- Handle the callback for this event;

- Delete listening events before disconnecting the reader/writer.

Note: The reader/writer triggers an event as a separate thread, which is not the main thread of the program.

  - Reader operation class

C#： Namespace: NetAPI.Protocol.VRP

JAVA： Package location: JavaAPI.Protocol.VRP

Android： Package location: rfid.uhfapi.protocol.vrp

Android y2005： Package location: rfid.uhfapi.y2005.protocol.vrp

This class encapsulates the four core operations of the reader/writer: connecting, disconnecting, sending data, and receiving data.

    - Explanation of Class Construction Methods

This chapter mainly explains the initialization of the reader/writer class.

- Constructor (Reader method)

Purpose: Instantiate the Reader class.

Grammar: public Reader(String readerName)

Parameters:

readerName： Reader/writer name, string.

remarks:

This constructor must be consistent with "2.4Configuration file (DeviceCpg. xml)Coordinated use. ReaderName is a custom string that remains unique in the configuration file. The constructor completes the assignment of connection parameters.

- Constructor (Reader method)

Purpose: Instantiate the Reader class.

Grammar: public Reader(String readerName, IPort commPort)

Parameters:

readerName： Reader/writer name, string.

commPort： Definition of communication interface.

remarks:

This constructor does not need to be used in conjunction with configuration files. The definition of readerName is the same as above. The constructor completes the assignment of connection parameters.

The communication interface IPort is an abstract class, and Rs232Port, Rs485Port, TcpClient Port, and HIDPort inherit from the Iport class to define several interfaces. These classes are located in the NetAPI namespace (JAVA: in the JavaAPI package)

The constructor is as follows:

public Rs232Port(String portName, BaudRate baudRate)

public Rs485Port(String portName, BaudRate baudRate, byte[] addresses)

public TcpClientPort(String ip, Int32 port)

Public HIDPort (string connStr)//USB communication

Parameters:

portName： Serial port name, string. For example: COM1

baudRate： BaudRate rate, BaudRate enumeration. The definition is as follows:

public enum BaudRate

{

R9600,//9600bps

R19200, //19200bps

R38400, //38400bps

R115200,//115200bps (factory default)

R230400//230400bps

}

Address: RS485 address list. RS485 address 1-255

ip： Reader IP. For example: 192.168.1.100

port： Reader network port. For example: 9090.

connStr: Connection information. The example is as follows

Rs232Port: ”COM3,11520”;

Rs485Port: “COM3,11520,2”;

TcpClientPort: “192.168.1.100:9090”;

HIDPort: ”1A86,E429”

remarks:

The Android version does not support RS485 and HID communication, and the Android y2005 version does not support TCP, RS485, and HID communication.

    - Description of Common Class Attributes

- Reader Name Attribute (ReaderName Member)

Purpose: Obtain the name of the reader/writer.

Grammar: public String ReaderName = "Device1";

remarks:

String type. Same as the readerName parameter in the constructor. The default is' Device1 '. If there are multiple reader/writer connections, please ensure that this name is not duplicated. Once specified in the constructor, it will automatically change here; It is strongly recommended that the reader/writer name be based on the definition of the constructor.

- Whether to connect flag attribute (IsConnected member)

Purpose: To obtain the connection flag.

Grammar:

C#：public bool IsConnected{get;}

JAVA：public Boolean getIsConnected();

remarks:

This member is of Boolean type. When using, be aware that abnormal disconnection of the reader may not be detected by the API in a timely manner.

- Reader/writer model attribute (ModelNumber member)

Purpose: To obtain the model of the reader/writer.

Grammar:

C#：public string ModelNumber { get; }

JAVA：public String getModelNumber();

remarks:

String type. The initial value is empty, and the member will only have a specific value after the reader/writer successfully connects.

- Does the tag data return antenna information attributes (IsEnabling Antenna member)

Purpose: Obtain whether the tag data returns the antenna information flag.

Grammar:

C#：public bool IsEnableAntenna { get; }

JAVA：public boolean getIsEnableAntenna();

- Does the tag data return the RSSI information attribute (IsEnable RSSI member)

Purpose: To obtain whether the tag data returns the echo intensity information flag.

Grammar:

C#：public bool IsEnableRSSI { get; }。

JAVA：public boolean getIsEnableRSSI();

- Communication Port Type Attribute (CommPort Member)

Purpose: To obtain the type of communication port.

Grammar:

C#：public IPort CommPort { get; set; }

JAVA：public IPort getCommPort();

JAVA：public void setCommPort(IPort value);

remarks:

It is strongly recommended to specify it in the constructor or configuration file, which is only used for queries.

- RS485 Reader/Writer Properties (RS485 Items Member)

Purpose: RS485 communication interface, this member is an RS485 Item class array representing the interface for each address.

Grammar: public RS485Item[] RS485Items;

Description: The RS485 Item class implements the following interfaces, with usage similar to the corresponding members or methods of the Reader class.

C#：

public byte Address；//  RS485 address

public FrequencyArea UhfBand = FrequencyArea.Unknown;

public string ModelNumber;

public bool IsEnableRSSI;

public bool IsEnableAntenna;

public bool Send(IHostMessage msg);

public bool Send(IHostMessage msg, Int32 timeout);

public bool Send(byte[] msgBuff);

JAVA：

public byte getAddress()；//  Obtain RS485 address

public byte setAddress()；//  Set RS485 address

public FrequencyArea UhfBand = FrequencyArea.Unknown;

public String getModelNumber();

public boolean getIsEnableRSSI();

public boolean getIsEnableAntenna();

public boolean Send(IHostMessage msg);

public boolean Send(IHostMessage msg, int timeout);

public boolean Send(byte[] msgBuff);

remarks:

The member is automatically generated after instantiation in the constructor of Reader. If it is not RS485 communication, the member is empty. The tag inventory data reception of this interface is the same as that of the Reader class, as shown in the OnInventoryReceived event of the Reader class.

- EPC filtering function (isEnabling EpcFilter method)

Purpose: EPC filtering function.

Grammar:

C#： Not currently supported

JAVA：public boolean isEnableEpcFilter();// Get whether EPC filtering function is enabled

public void setEnableEpcFilter(boolean enableEpcFilter);// Set whether to enable EPC filtering

public void epcFilterBuffClear();// Clear EPC filtering cache

remarks:

This function is implemented internally in the API and is used to filter EPC data uploaded by RFID read-write modules. If enabled, the same tag will only be uploaded once (this feature uses EPC cache comparison filtering. When the tag is uploaded for the first time and there is no tag record in the cache, the tag will be uploaded and saved in the cache table. When it is uploaded again, if the API comparison finds that the tag is already stored in the cache, it will not be uploaded). The upload interface is OnInventoryReceived of the Reader class. Clearing the cache and re instantiation of the Reader class can both re filter, but if not enabled, no filtering will be performed.

    - Explanation of Common Methods for Classes

This chapter explains the common operation methods of the Reader class.

- Establish connection (Connect method)

Purpose: Establish a communication connection with the reader/writer using the specified port, which is already specified in the constructor.

Grammar: public ConnectResponse Connect()

Return value:

Connection response, ConnectResponse is defined as follows

public class ConnectResponse

{

public bool IsSucessed = false;// Is the connection successful

public ErrInfo ErrorInfo;

}

Parameters:

ErrorInfo： Error message class, ErrInfo class; If there is a connection error, this class returns an error message.

Attention: Please power on the Y2006 device before connecting it. The specific interface is serialPortTool. getInstance(). openUhfDevice().

- Disconnect (Disconnect method)

Purpose: To disconnect ports that have already been connected.

Grammar: public void Disconnect()

Attention: After disconnecting the Y2006 device, please disconnect the power supply. The specific interface is serialPortTool. getInstance(). closeUhfDevice().

- Send message (Send method)

Purpose: To send messages.

Grammar: public bool Send(IHostMessage msg)

public bool Send(IHostMessage msg, int timeout)

Parameters:

msg： The specific type of message to be sent is IHostMessage.

timeout： Timeout, integer, in milliseconds.

Return value:

Whether the message was executed successfully, boolean type.

be careful:

  - The message that needs to be responded to is considered successful only when the message is sent successfully and a successful execution message is returned, otherwise it is considered a failure.

  - Messages that do not require a response will return success if the number of bytes sent is equal to the number of bytes to be sent.

  - The method without timeout parameter defaults to a timeout of 1000 milliseconds.

Note: For how to set whether a message requires a response, please refer to Chapter 3.

- Send message data (Send method)

Purpose: To send byte array message data.

Grammar: public bool Send(byte[] msgBuff)

Parameters:

msgBuff： The data to be sent is a byte array.

Return value:

Whether the message was successfully sent, boolean type.

remarks:

This method does not wait for a data response after sending data. Note: This method does not perform any data encapsulation or verification, but simply sends out the msgBuff content, which can be used for testing or custom message sending.

    - Class General Event Description

This chapter is about general reader/writer device events.

- Inventory message notification (OnInventoryReceived event)

Purpose: To receive tag reading data. In this API, loop reads of the MgTagInventory class and the MgTagRead class, as well as queries of the MgSlaveModeCacheQuery class, will trigger this event.

Grammar:

C#：public event InventoryReceivedHandle OnInventoryReceived;

C#：public delegate void InventoryReceivedHandle(string readerName, RxdTagData tagData);

JAVA：public EventHandler OnInventoryReceived;

Parameters:

readerName： Reader/writer name, the name specified in the constructor.

RxdTagData： Label data, defined as follows

C#:

public class RxdTagData

{

public byte Antenna{get;}

public byte Hub{get;}

public byte Port{get;}

public byte[] EPC{get;}

public byte[] TID{get;}

public byte[] User{get;}

public byte[] Reserved{get;}

public byte RSSI{get;}

public int Phase{get;}

public double GetRSSI{get;}

public DateTime UTC{get;}

}

JAVA:

public class RxdTagData

{

public byte getAntenna();

public byte getHub();

public byte getPort();

public byte[] getEPC();

public byte[] getTID();

public byte[] getUser();

public byte[] getReserved();

public byte getRSSI();

public int Phase{get;}

public double GetRSSI();

}

Antenna： Antenna number, 0 means this field was not returned

Hub:  Antenna splitter number, 0 indicates no return of this field, applicable to antenna splitter module

Port:  The branch antenna number corresponding to the antenna splitter, 0 indicates that this field has not been returned, applicable to the antenna splitter module

EPC： Tag EPC data, empty means this data was not returned

TID： Tag TID data, empty means this data was not returned

User： Label User data, empty means this data was not returned

Reserved： Label reserved data, empty means this data was not returned

RSSI： Echo intensity, 0 indicates that this field was not returned; Original value of echo intensity

GetRSSI： Converted echo intensity

Phase： Phase value, 0-180 degrees- 1 indicates that this data was not returned

UTC： Tag reading UTC time

remarks:

C # implements the InventoryReceptdHandle delegate to call back the reader/writer inventory message.

The OnInventoryReceived callback in JAVA contains two messages, one is the Reader class and the other is the RxdTagData class; JAVA calling steps:

1）Utils. Event msgNotify = new Utils.Event(this, "Reader_OnInventoryReceived");// Define callback method, this refers to the object where the callback method is located, depending on the specific implementation; Reader.InVenturyReceived is the name of the callback method; The event is in the Utils package.

2）reader. OnInventoryReceived.addEvent(msgNotify);// Load event callback

3) Implementation of event callback for void Reader-OInVenturyReceived (Reader sender, RxdTagData msg) {}//

- API Exception Notification (OnApiException Event)

Usage: This event is used to notify exceptions generated during API runtime.

Grammar:

C#：public static event ApiExceptionHandle OnApiException;

C#：public delegate void ApiExceptionHandle(String senderName, ErrInfo e);

JAVA：public static EventHandler OnApiException;

Parameters:

senderName： Reader/writer name, the name specified in the constructor.

ErrInfo： Error message class, ErrInfo class.

remarks:

C # implements ApiExceptionHandle delegation to callback exception messages.

The OnApiException callback in JAVA contains two messages, one is the senderName string and the other is the ErrInfo class; JAVA calling steps:

1）Utils. Event apiErrNotify = new Utils.Event(this, "Reader_OnApiException");// Define callback method, this refers to the object where the callback method is located, depending on the specific implementation; Reader.OnApiException is the callback method name; The event is in the Utils package.

2）Reader. OnApiException.addEvent(apiErrNotify);// Load event callback

3) Void Reader-OnApiException (String senderName, ErrInfo e) {}//Implementation of event callback

The vast majority of API exceptions are notified through this.

- Network disconnection notification (OnBrokenNetwork event)

Purpose: To receive notifications of reader/writer connection status.

Grammar:

C#：public event BrokenNetworkHandle OnBrokenNetwork;

C#：public delegate void BrokenNetworkHandle(string senderName, ErrInfo e);

JAVA：public static EventHandler OnBrokenNetwork;

Parameters:

senderName： Reader/writer name, the name specified in the constructor.

ErrInfo： Error message class, ErrInfo class.

remarks:

C # implements BrokenNetworkHandle delegation to callback reader/writer messages.

The OnBrokenNetwork callback in JAVA contains two pieces of information, one is the senderName string and the other is the ErrInfo class; JAVA calling steps:

1）Utils. Event brokenNetworkNotify = new Utils.Event(this, "Reader_OnBrokenNetwork");// Define callback method, this refers to the object where the callback method is located, depending on the specific implementation; Reader.OnBrokenNetwork is the name of the callback method; The event is in the Utils package.

2）Reader. OnBrokenNetwork.addEvent(brokenNetworkNotify);// Load event callback

3) Void Reader-OnBrokenNetwork (String senderName, ErrInfo e) {}//Implementation of event callbacks

- GPI input level status report event (OnGpiTriggerReceived event)

Purpose: GPI triggers status reporting.

Grammar:

C#：public event GpiTriggerHandle OnGpiTriggerReceived;

C#：public delegate void GpiTriggerHandle (string readerName, MsgGpiQuery info);

JAVA：public static EventHandler GpiTriggerHandle;

Parameters:

senderName： Reader/writer name, the name specified in the constructor.

Info: GPI input level status information, for the MgGpiQuery class, please refer to the received values in this class for details.

remarks:

C # implements GpiTriggerHandle delegation to callback reader/writer messages.

The GpiTriggerHandle callback in JAVA contains two pieces of information, one is the senderName string and the other is the info and sgGpiQuery classes; JAVA calling steps:

1）Utils. Event onGpiTriggerReceived = new Utils.Event(this, "Reader_GpiTriggerHandle");// Define callback method, this refers to the object where the callback method is located, depending on the specific implementation; Reader_GpiTriggerHandle is the name of the callback method; The event is in the Utils package.

2）reader. GpiTriggerHandle.addEvent(onGpiTriggerReceived);// Load event callback

3) Implementation of event callback for void Reader_GpiTriggerHandle (String senderName, MgGpiQuery info) {}//

- Report tag status event (OnTriggerInventoryState event)

Purpose: To receive notifications of tag reading status events from readers and writers.

Grammar:

C#：public event TriggerInventoryStateHandle OnTriggerInventoryState;

C#：public delegate void TriggerInventoryStateHandle (string readerName, bool isInventory);

JAVA：public static EventHandler OnTriggerInventoryState;

Parameters:

senderName： Reader/writer name, the name specified in the constructor.

isInventory： Boolean value, true is to start reading labels, false is to stop reading labels.

remarks:

C # implements TriggerInventoryStateHandle delegation to call back reader/writer messages.

The TriggerInventoryStateHandle callback in JAVA contains two pieces of information, one is the senderName string and the other is isInventory, a Boolean value; JAVA calling steps:

1）Utils. Event onTriggerInventoryState = new Utils.Event(this, "Reader_ OnTriggerInventoryState");// Define callback method, this refers to the object where the callback method is located, depending on the specific implementation; Reader_OnTriggerInventoryState is the name of the callback method; The event is in the Utils package.

2）reader. TriggerInventoryStateHandle.addEvent(onTriggerInventoryState);// Load event callback

3) Void Reader_OnTriggerInventoryState (String senderName, boolean isInventory) {}//Implementation of event callback

  - Message Instruction Base Class (AbstractHostMessage)

C#： Namespace: NetAPI.Protocol.VRP

JAVA： Package location: JavaAPI.Protocol.VRP

Android： Package location: rfid.uhfapi.protocol.vrp

Android y2005： Package location: rfid.uhfapi.y2005.protocol.vrp

The message base class actively sent by the upper computer. All message classes sent by the upper computer inherit from this class. The upper computer message is the basis for communication between the upper computer and the reader/writer, and all reader/writer actions are controlled through upper computer message instructions.

The upper computer message includes:

1) The message sent by the upper computer that requires a response from the reader/writer;

2) After the upper computer sends a message, there is no need to wait for a response from the reader/writer;

3) The upper computer responds to the message from the reader/writer.

The upper computer message class relationship diagram is shown in the following figure.

- Get or set whether a message needs to wait for a response (IsReturn member)

Purpose: Define an interface that is used to obtain or set whether a message requires a response, Boolean type.

Grammar:

C#：bool IsReturn { get; set; }

JAVA：boolean getIsReturn();

JAVA：void setIsReturn(boolean value);

remarks:

All messages in this API have already set this member by default. Unless there is a special need, do not change the value of this member.

- Retrieve message return status (Status member)

Purpose: To define an interface that is used to retrieve the status of message returns and to enumerate the type of 'MgStatus'.

Grammar:

C#：MsgStatus Status { get; set; }

JAVA：MsgStatus getStatus();

JAVA：void setStatus(MsgStatus value);

Return value:

The type status of the enum, including success, failure, and timeout status, is defined as follows

public enum MsgStatus

{

Timeout = -1,// timeout

Success = 0,// success

Failed=1//Failed

}

remarks:

- If the reader does not return within the specified timeout period, the status will be set to timeout upon return;

- When IsReturn is set to true, the reader returns a success message, and the status is set to successful when returned; If the reader/writer returns a message of failure, the status is set to 'failed' when returned;

- When IsReturn is set to false, if the upper computer successfully sends the message, the status will be set to successful upon return; If the upper computer fails to send a message, the status will be set as failed when returned.

- Return message content (ReceptdMessage member)

Purpose: To define an interface for obtaining message return content, which is determined by subclass information and will be described in the subclass.

Grammar:

C#：public ReceivedInfo ReceivedMessage { get; }

JAVA：public ReceivedInfo getReceivedMessage();

remarks:

Due to the different functions implemented by each subclass message, the content of the Received Info returned here cannot be determined and defaults to null.

- Return message error content (Error Info member)

Purpose: To define an interface for obtaining error information returned by messages. When the status of the message return is' failed ', the error information can be obtained through this member.

Grammar:

C#：ErrInfo ErrorInfo { get; }

JAVA：public ErrInfo getErrorInfo();

Return value:

ErrInfo type, including message content and error code, is defined as follows

public class ErrInfo

{

public String ErrMsg;//  Message content

public String ErrCode;//  error code

}

remarks:

Before use, you can check if it is null

  - Configuration file (DeviceCfg. xml)

The configuration file is an optional item, with the file name "DeviceCfg", an XML file, and is located in the same directory as the API. (Note: Android configuration files are located in the files directory under the package name, and the path can be obtained using getFilesDir())

The configuration file exists in the form of a reader/writer node, and its basic content includes:

1. Name of the reader/writer. (Reader name cannot be repeated)

2. Connection port type.

3. Connection string.

The configuration file format is as follows:

<?xml version="1.0" encoding="utf-8"?>

<Devices>

<LogName>Debug</LogName>

<LogLevel>Debug</LogLevel>  <PowerCalibration>CN[922.625],FCC2[921.25],FCC1[908.75],EU[866.9],STO1[815.0],STO2[890.0],STO3[965.0]</PowerCalibration>

<Device Name="Device1">

<Port Type="TcpClient">192.168.1.111:9090</Port>

<Address>2</Address>

</Device>

</Devices>where "Devices" is the root node.

The 'Device' node is a reader/writer configuration node. The 'Name' property is the reader/writer name, which is a custom string and should remain unique.

The 'Port' node is a communication port node, and its content is a connection string. The 'Type' attribute represents the port name. The connection string rules are as follows:

Example of serial port connection string: “COM1,115200”。 Among them, "COM1" is the serial port name, "115200" is the baud rate, separated by a half width comma.

TCP/IP protocol client connection string example: "192.168.1.100:9090". Among them, "192.168.1.100" is the IP address, "9090" is the port number, separated by a half colon in the middle.

The 'Address' node is an RS485 address with a value range of 1-255.

The "LogLevel" option is used to set the log level, which includes "Debug", "Info", "War", "Error", and "Fatal". The log level increases in order, and the default is "Info", which means that logs at the Debug level are not recorded, while logs above the Info level are recorded.

- Protocol API (Instruction Message) List

The message interface is divided into two categories, one is the reader message interface class (IReserMessage), and the other is the upper computer message interface class (IHostMessage). The messages mentioned below in this chapter, unless otherwise specified, are all inherited from the IHostMessage interface. For information on the message interface, please refer to2.3 Message Instruction Base Class (AbstractHostMessage)Section description.

Attention: Please refer to Appendix C for the interfaces supported by each version of the API.

  - Query and setting of device connection parameters

This chapter mainly focuses on querying and setting device connection related parameters.

    - Set/Query RS232 BaudRateConfig Class

Purpose: Set/query the baud rate of the RS232 communication interface on the application side.

- Constructor function

C #: public sgRs232BaudRateConfig ()//query

Public sgRs232BaudRateConfig (BaudRate baudRate)//Settings

JAVA: public sgRs232BaudRateConfig ()//query

Public sgRs232BaudRateConfig (BaudRate baudRate)//Settings

Parameters:

baudRate： BaudRate is defined in section 2.2.1

- Received Message

1) Baud rate

Purpose: To obtain baud rate.

C#：public BaudRate RS232BaudRate { get; }

JAVA：public BaudRate getRS232BaudRate();

Return value: Baud rate, defined as above.

remarks:

If the current connection is a serial port connection, then after changing the baud rate, it is necessary to reconnect with the changed baud rate.

    - Set/Query BaudRateConfig Class - For Some Old Devices

Purpose: Set/query the baud rate of the communication interface on the application side. This type is mainly targeted at some old devices, so it is recommended to use "3.1.1 Rs232 query/setting" for baud rate first, and then use this type after failure.

- Constructor function

C #: public sgBaudRateConfig ()//query

Public MgRBaudRateConfig (BaudRate baudRate)//Setting

JAVA： None

Parameters:

baudRate： BaudRate is defined in section 2.2.1

- Received Message

1) Baud rate

Purpose: To obtain baud rate.

C#：public BaudRate BaudRate { get; }

Return value: Baud rate, defined as above.

remarks:

If the current connection is a serial port connection, then after changing the baud rate, it is necessary to reconnect with the changed baud rate.

    - Set/Query RS485 BaudRateConfig Class

Purpose: To set/query the baud rate of the RS485 communication interface on the application end.

- Constructor function

C #: public sgRs485BaudRateConfig ()//query

Public Rs485BaudRateConfig (BaudRate baudRate)//Settings

JAVA: public sgRs485BaudRateConfig ()//query

Public Rs485BaudRateConfig (BaudRate baudRate)//Settings

Parameters:

baudRate： BaudRate is defined in section 2.2.1

- Received Message

1) Baud rate

Purpose: To obtain baud rate.

C#：public BaudRate RS485BaudRate { get; }

JAVA: public BaudRate getRS485BaudRate();

Return value: Baud rate, defined as above.

remarks:

If the current connection is a serial port connection, then after changing the baud rate, it is necessary to reconnect with the changed baud rate.

    - Set/query the RS-485 address of the local machine (using the sgRs485AutConfig class)

Purpose: Set/query the RS485 communication interface address (extension number) of the application terminal.

- Constructor function

C #: public sgRs485ArgeConfig ()//query

Public Rs485AutConfig (byte address)//Setting

JAVA: public sgRs485ArgeConfig ()//query

Public Rs485AutConfig (byte address)//Setting

Parameters:

address： Address, 1-255.

- Received Message

1) Address

Purpose: To obtain an address.

C#：public byte Address{ get; }

JAVA：public byte getAddress{ get; }

Return value: Address, 1-255.

    - Set/query the IP address of the network port (using the HttpIpAddressConfig class)

Purpose: Set/query the IP address of the application's network port.

- Constructor function

C #: public sgIpAddressConfig ()//query

Public HttpIpAddressConfig (string IP, string subnet, string gate)//Setting

JAVA: public sgIpAddressConfig ()//query

Public MgIpAddressConfig (String IP, String subnet, String gate)//Setting

Parameters:

IP: IP address, such as 192.168.1.100.

subnet： Subnet mask, such as 255.255.255.0.

gate： Gateway, such as 192.168.1.1.

- Received Message

1) IP address

Purpose: To obtain an IP address.

C#：public string IP { get; }

JAVA：public String getIP();

Return value: IP address, such as 192.168.1.100; The factory default is 192.168.1.100.

2) Subnet mask

Purpose: Obtain subnet mask.

C#：public string Subnet { get; }

JAVA：public String getSubnet();

Return value: Subnet mask, such as 255.255.255.0.

3) Gateway

Purpose: Obtain gateway.

C#：public string Gateway { get; }

JAVA：public String getGateway();

Return value: Gateway, such as 192.168.1.1.

    - Set/query the server/client mode of the network port (using the MgTcpModeConfig class)

Purpose: Set/query the network interface server/client mode of the application.

- Constructor function

C #: public sgTcpModeConfig ()//query

Public PGCTcpModeConfig (ReaderTcpMode, ushort ServerPortNO)//Setting

JAVA: public MgTcpModeConfig ()//query

Public MgTcpModeConfig (ReaderTcpMode mode, int ServerPortNO)//Setting

Parameters:

mode： The TCP connection mode on the reader/writer side is defined as follows.

public enum ReaderTcpMode

{

server,// Server mode (factory default configuration is server mode)

Client//Client mode

}

serverPortNO： The recommended service port for the reader/writer is between 1024 and 65535. The default is 9090, which only makes sense when the reader/writer is configured in server mode.

- Received Message

1) TCP connection mode on the reader/writer end

Purpose: To obtain the TCP connection mode of the reader/writer end.

C#：public ReaderTcpMode Mode { get; }

JAVA：public ReaderTcpMode getMode();

Return value: TCP connection mode on the reader/writer side, defined as above.

2) Reader/writer service port

Purpose: To obtain the reader/writer service port.

C#：public ushort ServerPortNO { get; }

JAVA：public int getServerPortNO();

Return value: Reader service port, recommended between 1024 and 65535. The default is 9090, which only makes sense when the reader/writer is configured in server mode.

- Instruction Description

The Ethernet communication provided by the reader/writer (based on TCP/IP protocol, only supports IPV4) includes two modes: the reader/writer as a server mode (also known as passive mode) and the reader/writer as a client mode (also known as active mode). At any given time, the reader/writer can only be in one of these two modes. The server of the reader/writer refers to the TCP connection initiated by an external device (such as a PC) to the reader/writer. The client mode of the reader/writer refers to the TCP connection initiated actively by the reader/writer to external devices.

    - Set/query whether DHCP is enabled on the network port (using the HttpDHCPModeConfig class)

Purpose: Set/query whether DHCP is enabled on the network port.

- Constructor function

C #: public cmd DHCPModeConfig ()//query

Public HttpDHCPModeConfig (byte mode)//Setting

JAVA：

Parameters:

mode： 0-not enabled (static IP), 1-enabled

- Set command response (Received Message)

1) Whether to enable: 0- Do not enable (static IP), 1- Enable

C#：public byte Mode {get;}

JAVA：

Return value: As described above

    - Query the MAC address of the network port (using the sgMacConfig class)

Purpose: To query the MAC address of the application's network port.

- Constructor function

C #: public sgMacConfig ()//query

JAVA: public sgMacConfig ()//query

- Received Message

1) Reader end MAC address

Purpose: To obtain the MAC address of the reader/writer end.

C #: public byte [] MAC {get;}//MAC address of the application side network port, 6 bytes.

public string MacStr();// MAC address string of application side network port

JAVA：public byte[] getMAC();// The MAC address of the application side network port, 6 bytes.

public String getMacStr();// MAC address string of application side network port

Return value: MAC address of the reader/writer end, defined as above.

    - Network port parameter reset command (MsDeviceResetByUdp class)

Usage: Reset device network parameters through UDP in device search network search. After resetting, the device defaults to server mode with an IP address of 192.168.1.100

- Constructor function

C#：public MsgDeviceResetByUdp(string deviceIP)

JAVA：

Parameters:

deviceIP： Reset the current IP address of the target device.

- Received Message

1) Device IP

Purpose: Return device IP

C#：public string DeviceIP{get;}

JAVA：

Return value: As described above.

    - Set/query the WIFI server/client mode on the application side (using the MsWifiModeConfig class)

Purpose: To set/query the WIFI server/client mode of the application.

- Constructor function

C #: public sgWifiModeconfig()//query

Public MongoDB WifiModConfig (ReaderTcpMode, ushort ServerPortNO)//Settings

JAVA: public sgWifiModeConfig ()//query

Public MongoDB WifiModConfig (ReaderTcpMode, int ServerPortNO)//Settings

Parameters:

mode： The TCP connection mode on the reader/writer side is defined as follows.

public enum ReaderTcpMode

{

server,// Server mode (factory default configuration is server mode)

Client//Client mode

}

serverPortNO： The recommended WIFI service port for the reader/writer application is between 1024 and 65535. The default is 9090, which only makes sense when the reader/writer is configured in server mode.

- Received Message

1) WIFI TCP connection mode for reader/writer application

Purpose: To obtain the WIFI TCP connection mode of the reader/writer application.

C#：public ReaderTcpMode Mode { get; }

JAVA：public ReaderTcpMode getMode();

Return value: WIFI TCP connection mode of the reader/writer application, defined as above.

2) WIFI service port on the reader/writer application end

Purpose: To obtain the WIFI service port of the reader/writer application.

C#：public ushort ServerPortNO { get; }

JAVA：public int getServerPortNO();

Return value: The WIFI service port on the reader/writer application side is recommended to be between 1024 and 65535. The default is 9090, which only makes sense when the reader/writer is configured in server mode.

- Instruction Description

The Ethernet communication provided by the reader/writer (based on TCP/IP protocol, only supports IPV4) includes two modes: the reader/writer as a server mode (also known as passive mode) and the reader/writer as a client mode (also known as active mode). At any given time, the reader/writer can only be in one of these two modes. The server of the reader/writer refers to the TCP connection initiated by an external device (such as a PC) to the reader/writer. The client mode of the reader/writer refers to the TCP connection initiated actively by the reader/writer to external devices.

    - Set/query the WIFI IP address on the application side (using the MsWifiIpAddressConfig class)

Purpose: Set/query the IP address of the application's WIFI.

- Constructor function

C #: public sgWifiIpAddressConfig ()//query

Public MongoDB WifiIpAddressConfig (string IP, string subnet, string gate)//Settings

JAVA: public sgWifiIpAddressConfig ()//query

Public MongoDB WifiIpAddressConfig (String IP, String subnet, String gate)//Settings

Parameters:

IP: IP address, such as 192.168.1.100.

subnet： Subnet mask, such as 255.255.255.0.

gate： Gateway, such as 192.168.1.1.

- Received Message

1) IP address

Purpose: To obtain an IP address.

C#：public string IP { get; }

JAVA：public String getIP();

Return value: IP address, such as 192.168.1.100; The factory default is 192.168.1.100.

2) Subnet mask

Purpose: Obtain subnet mask.

C#：public string Subnet { get; }

JAVA：public String getSubnet();

Return value: Subnet mask, such as 255.255.255.0.

3) Gateway

Purpose: Obtain gateway.

C#：public string Gateway { get; }

JAVA：public String getGateway();

Return value: Gateway, such as 192.168.1.1.

    - Set/query the WIFI hotspot parameters on the application side (using the MsWifiHotspotConfig class)

Purpose: Set/query WIFI hotspot parameters on the application side.

- Constructor function

C #: public sgWifiHotspotConfig ()//query

Public MongoDB WifiHotspotConfig (string ssid, string pwd)//Settings

JAVA: public sgWifiHotspotConfig ()//query

Public MongoDB WifiHotspotConfig (String ssid, String pwd)//Settings

Parameters:

SSID: WIFI hotspot name, supports ASCII code.

Pwd: WIFI hotspot password, supports ASCII code.

- Received Message

1) WIFI hotspot name

Purpose: To obtain the name of a WIFI hotspot.

C#：public string SSID{ get; }

JAVA：public String getSSID();

Return value: WIFI hotspot name, supports ASCII code.

2) WIFI hotspot password

Purpose: Obtain WiFi hotspot password.

C#：public string Password{ get; }

JAVA：public String getPassword ();

Return value: WIFI hotspot password, supports ASCII code.

- Instruction Description

Attention: When using a network connection, please ensure that the IP of the reader and the IP of the PC are in the same network segment.

    - Set/query the status of the WIFI network card on the application side (using the MsWifiStatusConfig class)

Purpose: To set/query the status of the WIFI network card on the application side.

- Constructor function

C #: public sgWifiStatusConfig ()//query

Public MgWifiStatusConfig (byte status)//Settings

JAVA: public sgWifiStatusConfig ()//query

Public MgWifiStatusConfig (byte status)//Settings

Parameters:

status： Parameter type. The meaning is as follows

00 means turn off WIFI device

01 indicates WIFI connection to the set network (client mode)/enabling WIFI hotspot (server mode)

02 indicates disconnection of WIFI from the set network connection (client mode)/disconnection of WIFI hotspot (server mode)

- Received Message

1) Network card status

Purpose: To obtain the status of the network card.

C#：public byte Status{ get; }

JAVA：public byte getStatus ();

Return value: WIFI network card status. The meaning is as follows

00 means WIFI is not turned on

01 indicates that WIFI has been connected to the set network (client mode)/WIFI hotspot has been turned on (server mode)

02 indicates that WIFI and the set network connection have been disconnected (client mode)/sWIFI hotspot has been turned off (server mode)

03 means WIFI cannot find the target network (client mode)

04 indicates that the WIFI access target network key is incorrect (client mode)

05 indicates abnormal WIFI

    - Query the MAC address of the WIFI on the application side (using the MsWifiMacConfig class)

Purpose: To query the MAC address of the application's WIFI.

- Constructor function

C #: public sgWifiMacConfig ()//query

JAVA: public sgWifiMacConfig ()//query

- Received Message

1) MAC address of WIFI on the application side of the reader/writer

Purpose: To obtain the MAC address of the WIFI on the reader/writer application end.

C #: public byte [] MAC {get;}//MAC address of the application side network port, 6 bytes.

public string getStringMAC();

JAVA：public byte[] getMAC();// The MAC address of the application side network port, 6 bytes.

public String getStringMAC();

Return value: MAC address of the reader/writer end, defined as above.

    - Set/query 4G server domain name/IP address (using the sgServerHostConfig class)

Purpose: Set/query server domain name/IP address.

- Constructor function

C #: public sgServerHostConfig ()//query

Public sgServerHostConfig (string domainName)//Setting

JAVA：

Parameters:

domainName： Domain/IP: Port number, for example "192.168.1.1:12345".

- Received Message

1) Domain/IP: Port Number

Purpose: Obtain domain name/IP: port number

C#：public string DomainName {get;}

JAVA：

Return value: As described above.

    - Set/query 4G access point APN (using the sgApnConfig class)

Purpose: Set/query 4G access point APN.

- Constructor function

C #: public sgApnConfig()//query

Public MgApnConfig (string apk)//Setting

JAVA：

Parameters:

apn： Set up access point APN.

- Received Message

1) Set Access Point APN

Purpose: Obtain APN settings for access points

C#：public string APN {get;}

JAVA：

Return value: As described above.

    - Query 4G IMEI (using the sg4GIMEイfig class)

Purpose: To query 4G IMEI.

- Constructor function

C #: public sg4GIMEFHIR ()//query

JAVA：

- Received Message

1) Set the IMEI of the access point

Purpose: Obtain IMEI

C#：public string IMEI {get;}

2) Set the IMSI of the access point

Purpose: Obtain IMSI

C#：public string IMSI {get;}

JAVA：

Return value: As described above.

    - Set/query the communication protocol type under the network port (using the sgNetProtocolType class)

Purpose:

- Constructor function

C #: public sgNetProtocolType()//query

Public sgNetProtocolType (ENetProtocolType type)//Setting

JAVA：

Parameters:

type： Communication type.

- Received Message

1) Protocol type

Purpose: Obtain protocol type

C#：public ENetProtocolType Type {get;}

The enumeration is as follows:

public enum ENetProtocolType

{

TCP,

HTTP,

MQTT

}

Return value: As described above.

    - Set/query MQTT connection authentication parameters (using the sgMQTTAuthorizeParam class)

Usage: Set/query the account and password used for authentication when MQTT connects to the server.

- Constructor function

C #: public sgMQTTAuthorizeParam()//query

Public sgMQTTAuthorizeParam (string acc, string pwd)//Setting

JAVA：

Parameters:

acc： account number. Cannot exceed 20 ASCII characters.

pwd： password. Cannot exceed 20 ASCII characters.

- Received Message

1) Accessing authenticated accounts

C#：public string Acc{get;}

2) Access authentication password

C#：public string Pwd{get;}

    - Set/Query MQTT Basic Parameters (MgMQTTBaseParam Class)

Purpose: Set/query basic parameters in MQTT communication

- Constructor function

C #: public sgMQTTBaseParam()//query

Public sgMQTTBaseParam (ushort interval, byte QoS, string publishTopic, string subscribeTopic)//Setting

JAVA：

Parameters:

interval： The interval time for publishing tag data is 100ms and should not exceed 65535.

QoS level, with values of 0, 1, and 2.

publishTopic： Publish a theme. Cannot exceed 32 ASCII characters.

subcribeTopic： Subscribe to the topic. Cannot exceed 32 ASCII characters.

- Received Message

1) Label data release interval

C#：public unshot Interval{get;}

2) QoS level

C#：public byte QoS{get;}

3) Publish Theme

C#：public string PublishTopic{get;}

4) Subscription Theme

C#：public string SubcribeTopic{get;}

    - Set/query wireless connection method (MongoDB Wireless CommunicationModeConfig)

Purpose: Set/query wireless connection methods (WIFI, Bluetooth, 4G)

- Constructor function

C #: public sgWireless CommunicationModeConfig ()//query

Public MongoDB Wireless CommunicationMode Config (EWireCommunicationMode type)//Setting

JAVA：

Parameters:

Type： Enumeration of connection methods. as follows

public enum EWirelessCommunicationMode

{

None,

WIFI,

BLE,

T_4G

}

- Received Message

1) Connection method

C#：public EWirelessCommunicationMode type{get;}

  - Equipment parameter query and setting

    - Query module software and hardware versions (using the MonsReaderVersions Query class)

Purpose: To query module product models and software and hardware versions.

- Constructor function

C#：public MsgReaderVersionQuery()

JAVA：public MsgReaderVersionQuery()

Parameter: None.

- Received Message

1) Product model

Purpose: To obtain the product model.

C#：public string ModelNumber { get; }

JAVA：public String getModelNumber();

2) Hardware version

Purpose: To obtain hardware versions.

C#：public string HardwareVersion{ get; }

JAVA：public String getHardwareVersion();

3) Software version

Purpose: To obtain software versions.

C#：public string SoftwareVersion { get; }

JAVA：public String getSoftwareVersion();

    - Query the software and hardware versions of the application processor (using the MonsCpuVersions Query class)

Purpose: To query module product models and software and hardware versions.

- Constructor function

C#：public MsgCpuVersionQuery()

JAVA：public MsgCpuVersionQuery()

Parameter: None.

- Received Message

1) Product model

Purpose: To obtain the product model.

C#：public string ModelNumber { get; }

JAVA：public string getModelNumber();

2) Hardware version

Purpose: To obtain hardware versions.

C#：public string HardwareVersion{ get; }

JAVA：public string getHardwareVersion();

3) Software version

Purpose: To obtain software versions.

C#：public string SoftwareVersion { get; }

JAVA：public string getSoftwareVersion();

    - Query the serial number of the read-write module product (using the sgReaderModulus SNQuery class)

Purpose: To query the serial number of the read-write module product.

- Constructor function

C #: public sgReaderModulus NQuery ()//Query

JAVA: public sgReaderModulus NQuery ()//Query

Parameter: None

- Received Message

1) Read and write module product serial number

Purpose: To obtain the serial number of the read-write module product.

C#：public string ReaderModuleSN { get; }

JAVA：publicString getReaderModuleSN()

Return value: Serial number format: "Year (4) Week (2) Serial number (4)", for example: "2021260001"。

- Instruction Description

The query here is for the serial number of the read-write module product.

    - Query the serial number of the reader/writer product (using the MonsReaderSNQuery class)

Purpose: To query the serial number of the reader/writer product.

- Constructor function

C #: public sgReaderSNQuery()//query

JAVA: Public MgReaderSNQuery()//Query

Parameter: None

- Received Message

1) Serial number of reader/writer product

Purpose: To obtain the serial number of the reader/writer product.

C#：public string ReaderSN { get; }

JAVA：public String getReaderSN()

Return value: Serial number format: "Year (4) Week (2) Serial number (4)", for example: "2021260001"。

- Instruction Description

The query here is the serial number of the CPU product for the reader/writer application.

    - Querying the RFID working status (using the sgRfidStatus Query class)

Purpose: To query the working status of RFID.

- Constructor function

C #: public sgRfidStatus Query()//Query

JAVA: public sgRfidStatus Query()//Query

Parameter: None.

- Received Message

1) Status

Purpose: To obtain the status of the reader/writer.

C#：public RfidStatus Status { get; }

JAVA：public RfidStatus getStatus();

Return value: Status enumeration, RfidStatus is defined as follows

public enum RfidStatus

{

Idle,//  00 indicates that the reader/writer is idle

Busy//01 indicates that the reader/writer is operating on the label

}.

2) Air interface protocol

Purpose: To obtain air interface protocols. Note: This section is only meaningful when the status is card reading.

C#：public AirProtocol Protocol { get; }

JAVA：public AirProtocol getProtocol();

Return value: enumeration of air interface protocols, AirProtocol defined as follows

public enum AirProtocol

{

ISO18000_6C,

ISO18000_6B,

ISO18000_6B_6C

}

3) Working frequency band

Purpose: To obtain the working frequency band.

C#：public FrequencyArea UhfBand { get; }

JAVA：public FrequencyArea getUhfBand();

Return value: working frequency band enumeration, with the definition of Frequency Area as follows

public enum FrequencyArea

{

CN,

FCC,

EU,

Unknown = -1

}

remarks:

CN (920MHz~925MHz)

FCC(902MHz~928MHz)

EU(865MHz~868MHz)

4) Antenna activation and power status

Purpose: To obtain antenna activation and power status.

C#：public AntennaPowerStatus[] Antennas { get; }

JAVA：public AntennaPowerStatus[] getAntennas();

Return value: Antenna enabled and power status. AntennaPowerStatus is defined as follows

public class AntennaPowerStatus

{

public byte AntennaNO;// Antenna number

public bool IsEnable;// Is it enabled

public byte PowerValue; // power value

}

remarks:

The unit of power value is dBm

    - Set/query the working status of the buzzer (using the MgReaderBeepConfig class)

Purpose: To set/query the working status of the buzzer.

- Constructor function

C #: public sgReaderBeepConfig()//query

Public SchoolReaderBeepConfig (boolean isBeep)//Settings

JAVA: Public MgReaderBeepConfig()//Query

Public SchoolReaderBeepConfig (boolean isBeep)//Setting

Parameters:

isBeep： Buzzer enabled, true is enabled, false is disabled.

- Received Message

1) Buzzer enabled status

Purpose: To determine if the buzzer is enabled.

C#：public bool IsBeep { get; }

JAVA：public bool getIsBeep()

Return value: true is enabled, false is disabled.

- Instruction Description

The buzzer here refers to the buzzer of the reader/writer.

    - Set/Query Reader WorkModeConfig Class

Purpose: Set/query the working mode of the reader/writer.

- Constructor function

C #: public sgReaderWorkModeConfig ()//query

Public sgReaderWorkModeConfig (WorkMode readerWorkMode, TagMB tagMB)//Settings

JAVA: Public MgReaderWorkModeConfig ()//Query

Public HttpReaderWorkModeConfig (WorkMode readerWorkMode, byte tagMB)//Settings

Parameters:

readerWorkMode： The working mode is defined as follows

public enum WorkMode

{

NormalMode,//  Normal mode

AgeingMode,//  AGING MODE

AutoScanMode//Automatic card reading mode

}

tagMB： Select the label area, defined in C # as follows

[Flags]

public enum TagMB

{

EPC = 0x01,

TID = 0x02,

User = 0x04

}

JAVA is of byte type, with values defined in C #, and can achieve the same functionality as C # through bitwise operations

- Received Message

1) Working mode

Purpose: To obtain the working mode.

C#：public WorkMode ReaderWorkMode { get; }

JAVA： not have

Return value: WorkMode enum type, obtains the working mode.

2) Select label area

Purpose: To obtain the selection label area.

C#：public TagMB TagMemoryBank { get; }

JAVA： not have

Return value: TagMB enumeration type, obtains the selected tag area.

- Instruction Description

When the upper computer sends a setting instruction to the reader/writer, selecting the label area is invalid when the working mode is normal mode or aging mode; When the working mode is automatic card reading mode, selecting the tag area is valid and can select to read EPC, TID, and USER.

    - Set/query the tag reading mode of the reader/writer (using the sgScanModeConfig class)

Purpose: Set/query the tag reading mode of the reader/writer.

- Constructor function

C #: public sgScanModeConfig ()//query

Public sgScanModeConfig (ScanMode scanMode)//Settings

JAVA： not have

Parameters:

scanMode： The tag reading mode is defined as follows:

public enum ScanMode

{

MutiTag,  //  Multi label mode

SingleTag,//  Quick read mode

DenseMode, //  Intensive reading mode

LowPower ,//  low power mode

EX10//Only supported on module models ending with "X"

}

- Received Message

1) Read label mode

Purpose: To obtain tag reading mode

C#：public ScanMode ReaderScanMode { get; }

JAVA： not have

Return value: ScanMode type, obtains the label reading mode.

    - Set/query sleep state (using the sgSleepStateConfig class)

Purpose: Set/query sleep status.

- Constructor function

C #: public sgSleepStateConfig ()//query

Public sgSleepStateConfig (boolean isSleep)//Setting

JAVA：

Parameters:

isSleep：false： Normal; true： Sleep.

- Received Message

1) Hibernation state

Purpose: To determine if it is in sleep mode

C#：public bool IsSleep{get;}

JAVA：

    - Set/query signal strength (RSSI) filtering threshold (using the sgRssiThresholdConfig class)

Purpose: To set/query the filtering threshold for Label Signal Strength (RSSI).

- Constructor function

C #: public sgRssiThresholdConfig ()//query

Public sgRssiThresholdConfig (byte rssi)//Setting

JAVA: public sgRssiThresholdConfig ()//query

Public sgRssiThresholdConfig (int rssi)//Setting

Parameters:

rssi： Label Signal Strength (RSSI) filtering threshold

- Received Message

1) Label Signal Strength (RSSI) filtering threshold

Purpose: To obtain the filtering threshold for Label Signal Strength (RSSI).

C#：public byte RSSI { get; }

JAVA：public int getRSSI();

Return value: Label Signal Strength (RSSI) filtering threshold.

- Instruction Description

The maximum RSSI filtering threshold is 255; When set to 0, it means that the RSSI filtering function is not enabled. When the value is greater than 0 and the RSSI value of the read tag is less than the set value, the reader will not upload the tag. During transparent transmission, the RSSI value is invalid.

    - Set/query duplicate tag filtering time (using the MgFilteringTimeConfig class)

Purpose: Set/query the filtering time for duplicate tags, that is, if a newly read tag has already been uploaded within the specified time, it will not be uploaded again. The tag filtering time only takes effect when the data upload method is set to time filtering.

- Constructor function

C #: public sgFilteringTimeConfig ()//query

Public MongoDB Filtering TimeConfig (ushort time)//Setting

JAVA: public sgFiltering TimeConfig ()//query

Public MgFilteringTimeConfig (int time)//Setting

Parameters:

time： Filter time, unit: 100ms. Value range: 0-65535; Used to specify the time interval for filtering duplicate data, 0 indicates no filtering.

- Received Message

1) Filter time

Purpose: To obtain filtering time.

C#：public ushort Time { get; }

JAVA：public int getTime();

Return value: Filter time, defined as above.

- Instruction Description

The duplicate tag filtering function is used to filter the duplicate tag data received by the reader/writer during the specified time cycle of card reading, reducing redundant data upload and reducing the overhead of data processing on the upper computer.

    - Set/query the interval for reading labels (using the MgIntervalTimeConfig class)

Purpose: Set/query the interval for reading labels.

- Constructor function

C #: public sgIntervalTimeConfig ()//query

Public MgIntervalTimeConfig (ushort readTime, ushort stopTime)//Setting

JAVA：public MsgIntervalTimeConfig()

public MsgIntervalTimeConfig(int readTime, int stopTime)

Parameters:

readTime： Reading label time, in milliseconds, maximum 65535ms

stopTime： Stop reading time, in milliseconds, maximum 65535ms

- Received Message

1) Read label time

Usage: Read label time, in milliseconds.

C#：public ushort ReadTime { get; }

JAVA：public int getReadTime();

Return value: Read label time, in milliseconds.

2) Stop reading time

Purpose: To obtain the stop reading time in milliseconds.

C#：public ushort StopTime { get; }

public int getStopTime();

JAVA:

Return value: Stop reading time, in milliseconds.

- Instruction Description

The setting/querying interval reading label function is used for the reader/writer to read labels intermittently. Specifically, it reads labels for a period of time (length of label reading time), then stops reading labels for a period of time (length of stop reading label time), does not read labels, reads labels again for a period of time, and then stops reading labels for a period of time, repeating the process.

    - Set/query idle waiting time (using the MsIdleTimeConfig class)

Purpose: Set/query idle waiting time.

- Constructor function

C #: public sgIdleTimeConfig ()//query

Public HttpIdleTimeConfig (ushort time)//Setting

JAVA: public FsIdleTimeConfig ()//query

Public HttpIdleTimeConfig (int time)//Setting

Parameters:

time： Idle waiting time, in milliseconds.

- Received Message

1) Idle waiting time

Purpose: To obtain idle waiting time.

C#：public ushort Time { get; }

JAVA：public ushort getTime();

Return value: Idle waiting time, in milliseconds.

    - Set/query heartbeat command configuration (using the MgKeepAliveConfig class)

Purpose: Set/query heartbeat commands.

- Constructor function

C #: public sgKeepAliveConfig ()//query

Public MongoDB KeepAliveConfig (boolean isEnable, ushort interval)//Setting

JAVA: public sgKeepAliveConfig ()//query

Public MongoDB KeepAliveConfig (boolean isEnable, int interval)//Setting

Parameters:

isEnable： Do you want to enable heartbeat command

interval： Heartbeat interval, 1~65535, in seconds.

- Received Message

1) Heartbeat switch

Purpose: To determine whether the heartbeat command is enabled.

C#：public bool IsEnable { get; }

JAVA：public boolean getIsEnable ();

Return value: Boolean type, whether to enable heartbeat command.

2) Heartbeat interval

Purpose: To obtain the heartbeat interval time.

C#：public ushort Interval { get; }

JAVA：public int getInterval ();

Return value: heartbeat interval, 1~65535, in seconds.

- Instruction Description

Used for confirming the connection status between the reader and the upper computer. The reader can send a heartbeat command to the upper computer, and the other party does not need to reply to the message after receiving it. When unlabeled data is uploaded, the reader sends a heartbeat command every set time; When tag data is uploaded, the reader/writer delays sending this message.

The heartbeat interval refers to the time interval between every two heartbeat commands, and the default heartbeat interval at the factory is 5 seconds.

Note: After configuring this command, the API will upload the disconnection status through the OnBrokenNetwork event, and there is no need to implement the heartbeat time judgment process again during secondary development. For information about the OnBrokenNetwork event, please refer to the relevant sections.

    - Set/query staged tag reading configuration parameters (using the sgReadTagCfgParamConfig class)

Purpose: Set/query tag reading configuration parameters.

- Constructor function

C #: public sgReadTagCfgParamConfig ()//query

Public sgReadTagCfgParamConfig (ReadTagCfgParamConfig)//Setting

JAVA: public sgReadTagCfgParamConfig ()//query

Public sgReadTagCfgParamConfig (ReadTagCfgParamConfig)//Setting

Parameters:

param： Read label configuration parameters. ReadTagCfgParam type parameter, defined as follows

public class ReadTagCfgParam

{

public Session Session_1; //  Session value in the first period

public Flag Flag_1;       //  Flag value in the first period

public byte Time_1;//  The duration of the first period, in seconds; 0 means not enabled

public Session Session_2; //  Session value in the second period

public Flag Flag_2;       //  Flag value in the second period

public byte Time_2;//  The duration of the second period, in seconds; 0 means not enabled

public byte LoopCount;//  Polling frequency; 0 is invalid

}

- Received Message

1) Read label configuration parameters

Purpose: To obtain tag reading configuration parameters.

C#：public ReadTagCfgParam ReadTagConfig { get; }

JAVA：public ReadTagCfgParam getReadTagConfig()

Return value: Same meaning as above.

Return value: As described above.

    - Set/query the working mode of the card issuer (using the sgTagInitConfig class)

Purpose: Set/query the working mode of the card dispenser. Mainly used for desktop readers and writers.

- Constructor function

C #: public sgTagInitConfig ()//query

Public sgTagInitConfig (byte RW, byte code, byte separator)//Setting

JAVA：

Parameters:

RW： 0-read-write, 1-read-only. In read-write mode, code and separator do not take effect

code：0- ASCII，1- HEX

separator： 0-Enter, 1-Space

- Set command response (Received Message)

1) Working mode: 0-read-write, 1-read-only

C#：public byte RW {get;}

JAVA：

Return value: As described above.

2) Working mode: 0-ASCII, 1-HEX

C#：public byte Code {get;}

JAVA：

Return value: As described above.

3) Working mode: 0-Enter, 1-Space

C#：public byte Separator {get;}

JAVA：

Return value: As described above.

    - Set/query UTC time (using the sgUteConfig class)

Purpose: Set/query UTC time.

- Constructor function

C #: public sgUteConfig()//query

Public MgUteConfig (DATE time)//Setting

JAVA: public sgUteConfig()//query

Public MgUteConfig (GregorianCalendar time)//Setting

Parameters:

Time: UTC time

- Received Message

1) UTC time

Purpose: To obtain the UTC time of the reader/writer.

C#：public DateTime UTC { get; }

JAVA：public GregorianCalendar getUTC();

Return value: UTC time.

- Instruction Description

Coordinated Universal Time, also known as Universal Time, World Standard Time, or International Coordinated Time. Due to the difference in abbreviations between English (CUT) and French (TUC), as a compromise, UTC is abbreviated.

Coordinated Universal Time is a time measurement system based on atomic time seconds, which strives to be as close as possible to Universal Time in terms of time. Chinese Mainland uses the ISO 8601-1988 Data Elements and Interchange Formats Information Interchange Date and Time Representation (GB/T 7408-1994) to call it the internationally coordinated time, replacing the original GB/T 7408-1994; Taiwan, China, China, adopts the Data Element and Exchange Format - Information Exchange - Date and Time Representation of CNS 7648, which is called Universal Time.

The time difference between UTC and Chinese Mainland, Hong Kong, Macao, Taiwan, China, Mongolia, Singapore, Malaysia, the Philippines, Western Australia is+8, that is, UTC+8.

The UTC time of this system is calculated starting from 1970-1-1.

    - Reader/writer reboot (using the sgAppCpuReset class)

Purpose: Restart the reader/writer.

- Constructor function

C#：public MsgAppCpuReset ()

JAVA：public MsgAppCpuReset ()

Parameters:

None.

- Received Message

The message has no further data to parse, and the Received Message is empty.

    - Restore factory settings (using the ` sgResetToFactoryDefault ` class)

Purpose: To restore factory default settings.

- Constructor function

C#：public MsgResetToFactoryDefault()

JAVA：public MsgResetToFactoryDefault()

Parameter: None.

- Received Message

The message has no further data to parse, and the Received Message is empty.

    - Set/query the maximum dwell time of the antenna (using the sgPortDwellMaxTimeConfig class)

Purpose: Set and query the maximum dwell time of the antenna. This function is applicable to X model modules.

- Constructor function

C #: public sgPortDwellMaxTimeConfig ()//query

Public sgPortDwellMaxTimeConfig (ushort maxTime)//Setting

Parameters:

matTime： Maximum dwell time, unit * 10ms.

- Received Message

- 1) Maximum dwell time of antenna

C#：public ushort MaxTime { get; }

    - Querying device temperature (using the FsTemperatureQuery class)

Purpose: To query the real-time temperature of the device, this function is applicable to the X model module.

- Constructor function

C #: public sgTemperatureQuery()//Query

- Received Message

- 1) Real time temperature of equipment

C#：public sbyteTemperature{ get; }

  - Equipment frequency band&antenna parameter query and setting

This chapter mainly covers equipment antenna information and parameters related to operating frequency bands and frequencies.

    - Query the number of antennas and power threshold of the reader/writer (using the MsReaderCapability Query class)

Purpose: To query the capabilities of the reader/writer.

- Constructor function

C#：public MsgReaderCapabilityQuery()

JAVA：public MsgReaderCapabilityQuery()

Parameters:

None.

- Received Message

1) Product model

Purpose: To obtain the product model.

C#：public string ModelNumber { get; }

JAVA：public string getModelNumber();

2) Number of antennas supported by the reader/writer

Purpose: To obtain the number of antennas supported by the reader/writer.

C#：public byte AntennaCount { get; }

JAVA：public byte getAntennaCount();

3) Minimum power supported by the reader/writer

Purpose: To obtain the minimum power supported by the reader/writer.

C#：public byte MinPowerValue { get; }

JAVA：public byte getMinPowerValue();

4) Maximum power supported by the reader/writer

Purpose: To obtain the maximum power supported by the reader/writer.

C#：public byte MaxPowerValue { get; }

JAVA：public byte getMaxPowerValue();

    - Set/query the working frequency band (using the sgUhfBandConfig class)

Purpose: Set/query working frequency band.

- Constructor function

C #: public sguhfBandConfig ()//query

Public MongoDB (Frequency Area band)//Setting

JAVA: public sguhfBandConfig ()//query

Public MongoDB (Frequency Area band)//Setting

Parameters:

band： The working frequency band, where indices [4-25] are only supported at the end of "X" for module models. The definition of Frequency Area is as follows

public enum FrequencyArea

{

CN,// (920MHz~925MHz)

FCC,// (902MHz~928MHz)

EU,// (865MHz~868MHz)

OPEN=3,// full band

JANPN_NoLBT_916_921=4,

JANPN_LBT_916_921 = 5,

JANPN_NoLBT_916_924 = 6,

KOREA=7,

CE_HIGH=8,

HK=9,

TAIWAN= 10,

MALAYSIA =11,

SOUTH_AFRICA=12,

BRAZIL=13,

THAILAND=14,

SINGAPORE=15,

AUSTRALIA=16,

INDIA=17,

URUGUAY=18,

VIETNAM=19,

ISRAEL=20,

PHILIPPINES=21,

INDONESIA=22,

NEW_ZEALAND=23,

PERU=24,

RUSSIA=25,

Unknown = -1

}

- Received Message

1) Working frequency band

Purpose: To obtain the working frequency band.

C#：public FrequencyArea UhfBand{ get; }

JAVA：public FrequencyArea getUhfBand();

Return value: Operating frequency band, defined as above.

    - Set/query the antenna to be used (in the MgAntennaConfig class)

Purpose: Set/query the antenna used.

- Constructor function

C #: public sgAntennaConfig ()//query

Public sgAntennaConfig (AntennaStatus [] antennaStatus Ary)//Setting

JAVA: public sgAntennaConfig ()//query

Public sgAntennaConfig (AntennaStatus [] antennaStatus Ary)//Setting

Parameters:

antennaStatusAry： List of antenna usage.

- Received Message

1) Antenna used

Purpose: To obtain the antenna used.

C#：public AntennaStatus[] AntennaStatusAry { get; }

JAVA：public AntennaStatus[] getAntennaStatusAry();

Return value: Antenna list, AntennaStatus type array.

remarks:

AntennaStatus class definition:

public class AntennaStatus

{

public byte AntennaNO;// Antenna number, value range 1-64

public bool IsEnable;// Is it enabled

}

explain:

Please enable the connected antenna according to the actual situation and prohibit the use of unconnected antennas to avoid damage to the RF module of the reader/writer.

    - Querying the connection status of antenna ports (using the sgAntennaStatusQuery class)

Purpose: To query the connection status of antenna ports.

- Constructor function

C #: public sgAntennaStatusQuery()//Query

JAVA: public sgAntennaStatusQuery()//Query

Parameters:

None.

- Received Message

1) Antenna port connection status

Purpose: To obtain the connection status of the antenna port.

C#：public AntennaConnectionStatus[] AntennaStatusAry{get;}

JAVA：public AntennaConnectionStatus[] getAntennaStatusAry();

Return value: Antenna connection status list, AntennaConnectionStatus type array.

remarks:

AntennaConnectionStatus class definition:

public class AntennaConnectionStatus {

public byte AntennaNO;// Antenna number

public boolean IsNormal;// True connection is normal, False connection exception

}

    - Set/query working frequency (using the MonsFrequent Config class)

Purpose: Set/query work frequency.

- Constructor function

C #: public sgFrequent Config ()//query

Public MongoDB Frequency Config (Frequency Table param)//Setting

JAVA: public sgFrequent Config ()//query

Public MongoDB Frequency Config (Frequency Table param)//Setting

Parameters:

param： The working frequency parameter is defined as follows

public class FrequencyTable

{

public bool IsAutoSet;// Automatic frequency setting

public byte[] FreqTable;// Frequency List

}

- Received Message

1) Working frequency

Purpose: To obtain working frequency.

C#：public FrequencyTable FreqInfo { get; }

JAVA：public FrequencyTable getFreqInfo();

Return value: Operating frequency, defined as above.

remarks:

There is a corresponding relationship between frequency list and frequency points. The definition of frequency list is as follows

public class UhfBandTable

{

public String[] List;//  Frequency point list

public UhfBandTable(Name ftName)

{

}

}

When the work frequency list FreqInfo is queried, its type is FrequenceTable, and the FreqTable value in this class is UhfBandTable The index value of a List, for example, if the value of an element in the FreqTable array is 0x03 and the current frequency band is CN, then the frequency point corresponding to 0x03 is UhfBandTable List[3]， Its frequency is 921.375MHz.

- Instruction Description

- When the frequency is set to true, the reader/writer automatically selects a frequency point within the RF band; When set to false, the reader does not automatically select a frequency point and can only use a frequency point based on the specified frequency list.

- Frequency list, used to specify the operating frequency points of the reader/writer in non automatic frequency selection mode. The frequency list is a list of channel numbers in the current operating frequency band. If three frequency points, 920.625, 922.375, and 924.375MHz, need to be specified in the national standard 920-925MHz frequency band, the frequency list should be {0,7,15}. The minimum number of frequency points is 1 and the maximum is 50.

- Frequency table:

| - name | - Scope (M) | - Stepping | - Frequency point |
| --- | --- | --- | --- |
| - National Standard (CN) | - 920.625 - 924.375 | - 0.25 | - 0x00 - 0x0F |
| - North America (FCC) | - 902.75 - 927.25 | - 1.5 | - 0x00 - 0x31 |
| - Europe (CE_LOW) | - 865.7 - 867.5 | - 0.6 | - 0x00 - 0x04 |
| - Full frequency band (OPEN) | - 780 - 1000 | - 5 | - 0x00 - 0x2C |
| - Japan (JANPN-NoLBT916_921) | - 916.8 - 920.4 | - 1.2 | - 0x00 - 0x03 |
| - Japan (JANPN_LBT-916_921) | - 916.8 - 918.0 | - 1.2 | - 0x00 - 0x05 |
| - Japan (JANPN-NoLBT916_924) | - 916.8 - 923.4 | - 1.2/0.2 | - 0x00 - 0x05 - 0x00 - 0x0C |
| - Korea (KOREA) | - 917.3 - 920.3 | - 0.6 | - 0x00 - 0x05 |
| - Europe (CE-HIGH) | - 916.3 - 918.7 | - 1.2 | - 0x00 - 0x02 |
| - Hong Kong (HK) | - 920.25 - 924.75 | - 0.5 | - 0x00 - 0x09 |
| - Taiwan, China, China (TAIWAN) | - 920.75 - 927.25 | - 0.5 | - 0x00 - 0x0D |
| - Malaysia (MALAYSIA) | - 919.25 - 922.75 | - 0.5 | - 0x00 - 0x7 |
| - South Africa (SOUTH_SFRICA) | - 915.6 - 918.8 | - 0.2 | - 0x00 - 0x10 |
| - Brazil (BRAZIL) | - 902.75 - 927.25 | - 0.5 | - 0x00 - 0x22 |
| - Thailand | - 920.25 - 924.75 | - 0.5 | - 0x00 - 0x0A |
| - Singapore (SINGAPORE) |  |  |  |
| - Australia (AUSTRALIA) |  |  |  |
| - India (INDIA) | - 865.1 - 966.9 | - 0.6 | - 0x00 - 0x3 |
| - Uruguay (URUGUAY) | - 916.25 - 927.25 | - 0.5 | - 0x00 - 0x16 |
| - Vietnam (VIETNAM) | - 918.75 - 922.25 | - 0.5 | - 0x00 - 0x07 |
| - Israel (ISRAEL) | - 916.25 |  | - 0x00 |
| - Philippines (PHILIPPINES) | - 918.25 - 919.75 | - 0.5 | - 0x00 - 0x03 |
| - Indonesia (INDONESIA) | - 917.25 - 921.75 | - 0.5 | - 0x00 - 0x03 |
| - New Zealand (NEW_ZEALAND) | - 922.25 - 926.75 | - 0.5 | - 0x00 - 0x09 |
| - Peru (PERU) | - 916.25 - 927.25 | - 0.5 | - 0x00 - 0x16 |
| - Russia (RUSSIA) | - 916.2 - 919.8 | - 1.2 | - 0x00 - 0x03 |

    - Set/query port power value (using the sgPowerConfig class)

Purpose: To set/query port power values.

- Constructor function

C #: public sgPowerConfig ()//query

Public sgPowerConfig (byte [] powers)//Settings

JAVA: public sgPowerConfig ()//query

Public sgPowerConfig (byte [] powers)//Settings

Parameters:

powers： Port power value, in antenna order, cannot be skipped, supports a maximum of 64 antennas; The power range depends on different readers and writers, and the number of antennas, minimum and maximum power values can be queried using the MsReaderCapacityQuery class.

- Received Message

1) Port power value

Purpose: To obtain port power values.

C#：public AntennaPower[] Powers { get; }

JAVA：public AntennaPower[] getPowers();

Return value: Port power value, defined as follows.

public class AntennaPower

{

public byte AntennaNO;

public byte PowerValue;

}

- Instruction Description

The power value is related to the card reading distance. The larger the power value, the farther the card reading distance, and the farther the card reading distance, the more likely it is to cause misreading; Conversely, the opposite is true.

    - Set/query port dwell time (using the sgPortDwellTimeConfig class)

Purpose: Set/query port dwell time.

- Constructor function

C #: public sgPortDwellTimeConfig (byte antNO)//query

Public sgPortDwellTimeConfig (ushort useTime, ushort minFreeTime, ushort maxFreeTime)//Settings

JAVA： not have

Parameters:

antNO： Antenna number

useTime： Usage time, indicating the working time of the specified antenna, in 10ms

minFreeTime： Minimum idle time, indicating the minimum idle time of the specified antenna, in units of 10ms

maxFreeTime： The maximum idle time refers to the minimum idle time of the specified antenna, measured in 10ms.

- Received Message

1) Usage time

Purpose: To obtain usage time.

C#：public ushort UseTime { get; }

JAVA： not have

Return value: ushort type, get usage time.

2) Minimum idle time

Purpose: To obtain the minimum idle time.

C#：public ushort MinFreeTime { get; }

JAVA： not have

Return value: Ushort type, obtains the minimum idle time in 100 milliseconds.

3) Maximum idle time

Purpose: To obtain the maximum idle time.

C#：public ushort MaxFreeTime { get; }

JAVA： not have

Return value: Ushort type, obtains the maximum idle time in 100 milliseconds.

- Instruction Description

1) When the maximum idle time is 0, the usage time is the maximum residence time.

2) When configuring, the antenna number is not included, and all antennas are configured uniformly

    - Set/query the power value for reading labels on a certain port (using the ` sgReadPowerConfig ` class)

Purpose: Set/query the power value for reading labels on a certain port.

- Constructor function

C #: public sgReadPowerConfig ()//query

Public MongoDB ReadPowerConfig (byte antennaNO, byte power)//Setting

JAVA: public sgReadPowerConfig ()//query

Public MongoDB ReadPowerConfig (byte antennaNO, byte power)//Setting

Parameters:

antennaNO： Antenna number; It refers to the ability to individually set and query the tag reading output power of a certain antenna;

power： Read label output power (dBm): range 0-33dBm.

- Received Message

1) Port number

Purpose: Obtain port number.

C#：public byte AntennaNO { get; }

JAVA：public byte getAntennaNO ();

Return value: Port power value, defined as follows.

public class AntennaPower

{

public byte AntennaNO;

public byte PowerValue;

}

2) Port power value

Purpose: To obtain port power values.

C#：public byte Powers { get; }

JAVA：public byte getPowers();

Return value: Port power value.

- Instruction Description

This command allows for the individual setting and querying of the read tag power value for a certain antenna port; And the PGCPowerConfig command can batch set and query the read tag power values of all antenna ports.

    - Set/query the power value for writing tags on antenna ports (using the MgWritePowerConfig class)

Purpose: Set/query the power value of antenna port label writing.

- Constructor function

C #: public sgWritePowerConfig ()//query

Public WritePowerConfig (byte power)//Settings

JAVA: public sgWritePowerConfig ()//query

Public WritePowerConfig (byte power)//Settings

Parameters:

power： Port power value, all antenna ports have only one unified output power value for label writing; Range 9-33dBm

- Received Message

1) Port power value

Purpose: To obtain port power values.

C#：public byte Power { get; }

JAVA：public byte getPower();

Return value: Port power value.

  - Equipment Gen 2 parameter query and setting

This chapter mainly focuses on querying and setting parameters related to Gen 2.

    - Set/query supported air interface protocols (such as the MsAirProtocolConfig class)

Purpose: To set/query supported air interface protocols.

- Constructor function

C #: public sgAirProtocolConfig ()//query

Public sgAirProtocolConfig (AirProtocolparam)//Settings

JAVA: public sgAirProtocolConfig ()//query

Public sgAirProtocolConfig (AirProtocolparam)//Settings

Parameters:

param： The air interface protocol is defined as follows

public enum AirProtocol

{

ISO18000_6C,//  Indicates support for 6C protocol labels, set as default parameters

ISO18000_6B,//  Indicates support for 6B protocol labels

ISO18000_6B_6C//indicates support for 6B+6C protocol labels

}

- Received Message

1) Air interface protocol

Purpose: To obtain supported air interface protocols.

C#：public AirProtocol Protocol { get; }

JAVA：public AirProtocol getProtocol();

Return value: Air interface protocol, defined as above.

    - A sets/queries the encoding mode of the communication protocol (using the sgCommunicationModeConfig class)

Purpose: Set/query supported communication protocol encoding modes. Models ending in "D" support index [9-12], models ending in "X" support index [13-23], and other models support index [0-4].

- Constructor function

C #: public sgCommunicationModeConfig ()//query

Public MgCommunicationModeConfig (byte mode)//Setting

Java:

Parameters: enumerated as follows

0 Tari=25us, FM0, LHF=40KHz (IMPINJ R2000/Fudan Micro)

1，  Tari=25us, Miller 4, LHF=250KHz (IMPINJ R2000/Fudan Micro)

2，  Tari=25us, Miller 4, LHF=300KHz (IMPINJ R2000/Fudan Micro)

3，  Tari=6.25us, FM0, LHF=400KHz (IMPINJ R2000/Fudan Micro)

4，  Tari=25us, Miller2, LHF=320KHz (IMPINJ R2000/Fudan Micro)

9,   Tari=12.5us, FM0, LHF=160KHz (only supported by YRM8XXD)

10,   Tari=12.5us, Miller8, LHF=160KHz (only supported by YRM8XXD)

11,   Tari=12.5us, FM0, LHF=250KHz (only supported by YRM8XXD)

12,   Tari=6.25us, Miller 4, LHF=320KHz (only supported by YRM8XXD)

- Tari=20us，Miller8，LHF=160KHz，PIE=2  (E710/E510/E310)

- Tari=20us，Miller4，LHF=250KHz，PIE=2  (E710/E510/E310)

- Tari=20us，Miller4，LHF=320KHz，PIE=2  (E710/E510/E310)

- Tari=20us，Miller2，LHF=320KHz，PIE=2  (E710/E510/E310)

- Tari=15us，Miller2，LHF=320KHz，PIE=2  (E710/E510/E310)

- Tari=7.5us，Miller4，LHF=640KHz，PIE=2  (E710/E510)

- Tari=7.5us，Miller4，LHF=640KHz，PIE=1.5 (E710/E510)

- Tari=7.5us，Miller2，LHF=640KHz，PIE=2  (E710/E510)

- Tari=6.25us，Miller2，LHF=640KHz，PIE=1.5(E710/E510)

- Tari=7.5us，Miller0，LHF=640KHz，PIE=2  (E710)

- Tari=6.25us，Miller0，LHF=640KHz，PIE=1.5(E710)

    - Set/query communication WiegandParameterConfig class

Purpose: Set/query supported Wiegand parameters.

- Constructor function

C #: public sgWiegandParameterConfig ()//query

Public sgWiegandParameterConfig (WiegandParameter param)//Setting

Java:

Parameters:

public class WiegandParameter

{

public bool IsEnable; // Is it enabled

public WiegandMode CommunicationMode;// parameter mode

public WiegandTransmissionDataType TransmissionDataType;// Data area (EPC, TID)

}

    - Set/query the starting Q value (using the sgQValueConfig class)

Purpose: Set/query the starting Q value.

- Constructor function

C #: public sgQValueConfig()//query

Public MgQValueConfig (byte q)//Setting

JAVA：

Parameters:

q： Specify the time slot value for antenna configuration, with a valid range of 0-15

- Received Message

1) Q value

Purpose: To obtain Q value.

C#：public byte Q{ get; }

JAVA：

Return value: Time slot value for antenna configuration.

    - Set/query the Session of the port (using the MsSessionConfig class)

Purpose: To set/query sessions for ports.

- Constructor function

C #: public sgSessionConfig ()//query

Public FsSessionConfig (SessionInfo param)//Setting

JAVA：

Parameters:

param： Inventory parameter, SessionInfo type parameter, defined as follows

public class SessionInfo

{

public Session Session;

public Flag Flag;

}

session：S0, S1,S2,S3

Flag Inventory Flag:

Flag_S represents inventory using only Flag A;

Flag_S represents inventory using only Flag B;

FlagA_B represents alternating use of Flag A and Flag B double-sided inventory;

- Received Message

1) Inventory information

Purpose: To obtain inventory information configuration.

C#：public SessionInfo SessionParam { get; }

JAVA：

Return value: SessionInfo type parameter, as above.

    - Set/query TagFocus options (using the sgTagFocusConfig class)

Purpose: Set/query TagFocus options.

- Constructor function

C #: public sgTagFocusFigure()//query

Public sgTagFocusConfig (boolean isEnable)//Setting

JAVA: public sgTagFocusFigure()//query

Public sgTagFocusConfig (boolean isEnable)//Setting

Parameters:

IsEnable: true means enable TagFocus feature, false means disable TagFocus feature

- Received Message

1) Do you want to enable TagFocus function

Purpose: To obtain the option to enable TagFocus functionality.

C#：public bool IsEnable { get; }

JAVA：public boolean get IsEnable ();

Return value: Boolean value, true indicates enabling TagFocus function, false indicates disabling TagFocus function.

    - Set/query FastID options (using the sgFastIDConfig class)

Purpose: Set/query TagFocus options.

- Constructor function

C #: public sgFastIDconfig()//query

Public sgFastIDConfig (boolean isEnable)//Setting

JAVA: public sgFastIDconfig()//query

Public sgFastIDConfig (boolean isEnable)//Setting

Parameters:

IsEnable: true means FastID feature is enabled, false means FastID feature is disabled

- Received Message

1) Do you want to enable FastID function

Purpose: To obtain the option to enable FastID functionality.

C#：public bool IsEnable { get; }

JAVA：public boolean get IsEnable ();

Return value: Boolean value, true indicates enabling FastID function, false indicates disabling FastID function.

  - GPIO parameter query and setting

This chapter mainly focuses on GPIO input/output queries and settings.

    - Query GPIO interface definition (MgGpioDefinitionQuery class)

Purpose: The reader/writer defines four external IO ports and specifies the GPIO port numbers that need to be set/queried. 1 represents GPO1, 2 represents GPO2, 3 represents GPO3, and 4 represents GPO4. The number and definition of specific devices may vary.

- Constructor function

C #: public sgGpioDefinitionQuery()//query

JAVA: Public MgGpioDefinitionQuery()//Query

Parameter: None.

param： The interface definition enumeration is defined as follows.

- Received Message

1) GPIO interface definition

Purpose: To obtain GPIO interface definitions.

C#：public GpioInfo[] GpioInformation { get; }

JAVA：public GpioInfo[] getGpioInformation();

Return value: GPIO interface definition information array, where Gpio class is defined as follows.

public class GpioInfo

{

public byte PortNO;

public GpioDefinition Definition;

}

public enum GpioDefinition:byte

{

GPI,//IO input

GPO,//IO output

Disabled=0xff//Disabled

}

    - Query GPIO input voltage level (using the sgGpiQuery class)

Purpose: To query GPIO output level. The reader/writer defines four external output ports and specifies the GPIO port number that needs to be configured. 1 represents GPO1, 2 represents GPO2, 3 represents GPO3, and 4 represents GPO4.

- Constructor function

C #: Public MgGpiQuery (byte portNO)//Query

JAVA: Public MgGpiQuery (byte portNO)//Query

Parameters:

PortNO: GPO port number.

Param: GPO level parameter. Defined as follows

public class GpioLevelParameter

{

public byte PortNO; //  GPO port number

public GpioLevel Level;// level

}

public enum GpioLevel

{

Low, // Low Level

High//High level

}

- Received Message

1) GPI level parameters

Purpose: To obtain GPI level parameters.

C#：public GpioLevelParameter GpiLevel { get; }

JAVA：public GpioLevelParameter getGpiLevel();

Return value: GPI level parameter, defined as above.

    - Set GPIO output level (using the sgGpoConfig class)

Purpose: Set GPIO output level. The reader/writer defines four external IO ports and specifies the GPIO port number that needs to be set. 1 represents GPO1, 2 represents GPO2, 3 represents GPO3, and 4 represents GPO4.

- Constructor function

C #: Public MgGpoConfig (GpioLevelParameter param)//Setting

JAVA: Public MgGpoConfig (GpioLevelParameter param)//Setting

Parameters:

PortNO: GPO port number. The reader/writer defines two external output ports (GPO3, GPO4), specifying the GPIO port number that needs to be configured, with 3 representing GPO3 and 4 representing GPO4.

Param: GPO level parameter. Defined as follows

public class GpioLevelParameter

{

public byte PortNO;//  GPO port number

public GpioLevel Level;// level

}

public enum GpioLevel

{

Low,// Low Level

High//High level

}

- Received Message

The message has no further data to parse, and the Received Message is empty.

    - Set/Query GPIO Input Triggered Label Operation (MgGpiTriggerConfig Class)

Purpose: To set/query tag operations triggered by GPIO inputs.

- Constructor function

C #: Public MgGpiTriggerConfig (byte portNO)//Query

Public MgGpiTriggerConfig (GpiTriggerParameter param)//Setting

JAVA: Public MgGpiTriggerConfig (byte portNO)//Query

Public MgGpiTriggerConfig (GpiTriggerParameter param)//Setting

Parameters:

PortNO: Gpi port number.

Param： Trigger parameters. Defined as follows

public class GpiTriggerParameter

{

public byte PortNO;//  Specify the port number to be configured, where 1 represents GPO1 and 2 represents GPO2

public GpiTriggerCondition TriggerCondition;// Trigger condition

public GpiStopCondition StopCondition;// Stop condition

public ushort DelayTime;// Delay time, specifying the delay time as the stop condition

public IHostMessage TriggerMsg;// Trigger message, use null for empty instruction

}

public enum GpiTriggerCondition

{

Disabled,//  Indicates the disabling of GPIO triggering function for the current port;

HighLevel,//  Indicates GPIO high-level triggering;

LowLevel,// Indicates GPIO low level triggering;

RisingEdge,//  Indicates GPIO rising edge triggering;

FallingEdge//indicates GPIO falling edge triggering;

}

public enum GpiStopCondition

{

StateInversion,// Using GPIO state inversion as the stop condition (opposite to the trigger condition, high level corresponds to low level);

Delayed/using delay as the stopping condition;

}

- Received Message

1) GPIO trigger parameters

Purpose: To obtain GPIO trigger parameters.

C#：public GpiTriggerParameter TriggerParameter{ get; }

JAVA：public GpiTriggerParameter TriggerParameter();

Return value: GPIO trigger parameter, defined as above.

- Instruction Description

The GPIO of the reader/writer implements a GPIO event triggering mechanism on the basis of basic input/output functions, and the GPIO event triggering mechanism can be easily linked with other devices.

The basic principle of GPIO event triggering mechanism is to bind the GPIO input state with the relevant operation instructions of the reader/writer, Different input states of GPIO can trigger responsive reader/writer operations. Each GPIO port can only be bound to one pair of reader/writer instructions, and each GPIO port can be bound separately.

This function is generally used for GPIO to trigger the corresponding RFID card reading operation. After meeting the stop condition, the reader/writer automatically stops reading the card.

  - Label operation

    - Stop reading tags (using the MonsPowerOff class)

Purpose: Turn off the power amplifier. Frequently used and stopped reading.

- Constructor function

C #: public sgPowerOff ()//Turn off the amplifier, default not waiting for return

JAVA: public sgPowerOff ()//Turn off the amplifier, default does not wait for return

Parameter: None.

- Received Message

The message has no further data to parse, and the Received Message is empty

    - Set/query 6C tag to read containing fields (using the sg6CTagFieldConfig class)

Purpose: To set/query fields for reading 6C label data.

- Constructor function

C #: public sg6CTagFieldConfig ()//query

Public Mg6CTagFieldConfig (boolean isEnabling Antenna, boolean isEnabling RSSI)//Setting

Public Mg6CTagFieldConfig (boolean isEnabling Antenna, boolean isEnabling RSSI, boolean isEnabling Phase)//Setting

JAVA: public sg6CTag FieldConfig ()//query

Public Mg6CTagFieldConfig (boolean isEnabling Antenna, boolean isEnabling RSSI)//Setting

Parameters:

isEnableAntenna： Does it include an antenna field.

isEnableRSSI： Does it contain an RSSI field.

isEnablePhase： Does it include a phase field.

- Received Message

1) Does it include an antenna field

Purpose: Obtain whether the antenna field is included.

C#：public bool IsEnableAntenna

JAVA：public boolean getIsEnableAntenna();

Return value: Whether the antenna field is included. false： The antenna port indicating that the data frame does not contain a receiving tag; true： Indicate the antenna port containing the receiving tag in the data frame;

Note: The antenna port field is only applicable for reading tag data

2) Does it include an RSSI field

Purpose: Obtain whether the RSSI field is included.

C#：public bool IsEnableRSSI

JAVA：public boolean getIsEnableRSSI();

Return value: Whether the RSSI field is included. false： The RSSI value indicating that the data frame does not contain a receiving tag; true： Indicates the RSSI value of the received tag in the data frame.

    - 6C Tag Inventory (MgTagInventory class)

Purpose: Read 6C tag EPC, tag inventory.

- Constructor function

C #: public sgTagInventory()//Scan tag EPC, unlimited time, unlimited times

Public sgTagInventory (InventoryTagParameter param)//Scan tag EPC

JAVA: public sgTagInventory()//Scan tag EPC, unlimited time, unlimited times

Public sgTagInventory (InventoryTagParameter param)//Scan tag EPC

Parameters:

param： Scan label parameters, defined as follows

public class InventoryTagParameter

{

public ushort ReadCount;// Read frequency

public ushort TotalReadTime;// Read total time

public ushort TagFilteringTime;// Duplicate label filtering time

public ushort ReadTime;//  Intermittent reading time, used in conjunction with StopTime

public ushort StopTime;//  Intermittent stop time, used in conjunction with ReadTime

public ushort IdleTime;//  Idle waiting time

public TagParameter SelectTagParam = null;//  Tag selection parameters

}

public class TagParameter

{

public MemoryBank MemoryBank;// Label data area

public UInt32 Ptr;// Starting address of label

public byte[] TagData;// labeled data

}

MemoryBank： Label data area. Attention: Do not specify a reserved area when selecting labels, and do not specify a TID area when writing labels.

Ptr： The starting address of the label, as the size of each data area of the label from different manufacturers may vary, the range of the starting address is also different, and cannot exceed the maximum value of the matching data area of the operated label. Please refer to the data manual of the corresponding label for specific numerical values. If the maximum value of the label data area cannot be determined, you can try reading the operation bank of the label before operation to confirm the maximum value of the matching data area. Note: The starting address of the selection label parameter is in bits; The starting address for reading and writing label parameters is in words.

public enum MemoryBank

{

ReservedMemory,//  Reserved area

EPCMemory,//EPC Data Area

TIDMemory,//TID data area

UserMemory//User Data Area

}

ReadCount： Set the required number of times to read, and stop reading the card after reaching the set number of times. A value of 0 indicates an unlimited number of times.

TotalReadTime： Set the required reading time, in 10ms, and stop reading the card after reaching the set time. A value of 0 indicates no time limit.

- Received Message

1) Label data

Purpose: To obtain tag data, which only includes EPC. In addition, there may be antenna number and RSSI data, depending on the configuration of Mg6CTagFieldConfig.

C#：public RxdTagData TagData {get;}

JAVA：public RxdTagData getTagData();

Return value: Label data. Please refer to the OnInventoryReceived section for instructions on RxdTagData.

remarks:

Only when the number of reads is equal to 1, the tag data is directly returned in the response of this message. Cyclic card reading only returns when tag data is read, without any failure message, and the tag data is returned to the OnInventoryReceived event of the Reader class.

- Instruction Description

This function is used to read the EPC code of the label, also known as inventory. The EPC code is used to identify the tracked item in practical applications, and its function is similar to the barcode of a product. The EPC code is the easiest label data to obtain.

    - 6C tag universal reading (sgTagRead class)

Purpose: Universal reading of 6C labels.

- Constructor function

C #: public sgTagRead()//Scan tag TID, unlimited time, unlimited times

Public sgTagRead (ReadTagParameter param)//Scan tags

JAVA: public sgTagRead()//Scan tag TID, unlimited time, unlimited times

Public sgTagRead (ReadTagParameter param)//Scan tags

Parameters:

param： Read tag parameters, inherited from InventoryTagParameter, note that they have the same parameters as InventoryTagParameter, see InventoryTagParameter description. ReadTagParameter is defined as follows

public class ReadTagParameter : InventoryTagParameter

{

public bool IsLoop = false;

public byte[] AccessPassword = new byte[4];

public bool IsReturnEPC = false;

public bool IsReturnTID = false;

public UInt32 UserPtr;

public byte UserLen;

public bool IsReturnReserved = false;

new public ushort ReadCount = 1;//  Note: This is read once by default

new public ushort ReadTime = 100;

}

IsLoop：false, In single read mode, the reader/writer performs one round of card reading operation on each enabled antenna to end the card reading operation and automatically enter an idle state. true, Indicates continuous reading mode, where the reader/writer continues to read the card until it receives a stop command and ends the card reading operation.

AccessPassword： Tag access password, 4 bytes, default all zeros.

IsReturnEPC： Specify whether to return EPC data. False means no need to return, true means need to return.

IsReturnTID： Specify whether to return TID data. False means no need to return, true means need to return. Explanation: When reading the TID code of tags, the TID codes of mainstream manufacturers are currently non rewritable and have unique encoding. Therefore, TID codes are often used as unique identifiers in applications. The length of TID codes for different models of tag chips may vary, and this function can adapt to TID code lengths ranging from 32 to 112 bits.

UserPtr： The first address of the user area.

UserLen： The length of the user area, measured in words, ranges from 0 to 127, with 0 indicating no return of User data.

IsReturnReserved： Specify whether to return the reserved area data. The content of the reserved area data is: destruction password 2 words+access password 2 words

ReadCount： ditto.

ReadTime： ditto.

- Received Message

1) Label data

Purpose: To obtain tag data, which may include antenna number and RSSI data, depending on the configuration of Mg6CTagFieldConfig.

C#：public RxdTagData TagData {get;}

JAVA：public RxdTagData getTagData();

Return value: Label data. Please refer to the OnInventoryReceived section for instructions on RxdTagData.

remarks:

Only when the number of reads is equal to 1, the tag data is directly returned in the response of this message. Loop the card tag data and return it to the OnInventoryReceived event of the Reader class.

    - 6C Tag Selection (MgTagSelect class)

Purpose: Select the 6C label for operation.

- Constructor function

C#：public MsgTagSelect(TagParameter param)

JAVA：public MsgTagSelect(TagParameter param)

Parameters:

param： Select label parameters and define them as above. Note: The starting address is in bits; The matching area cannot be selected as a reserved area.

- Received Message

The message has no further data to parse, and the Received Message is empty.

    - 6C Tag Writing (MgTagWrite class)

Usage: Write 6C tags.

- Constructor function

C#：public MsgTagWrite(WriteTagParameter param) //

JAVA：public MsgTagWrite(WriteTagParameter param)

Parameters:

param： Write tag parameters, defined as follows

public class WriteTagParameter

{

public bool IsLoop = false;

public byte[] AccessPassword = new byte[4];

public TagParameter SelectTagParam = null;

public TagParameter[] WriteDataAry = null;

}

IsLoop：false， Indicates a single write mode, which only performs write operations on specified tags, and ends the write operation after the write operation is completed. true， Indicates a continuous write mode, where multiple tags that meet the matching rules are continuously written until the power amplifier shutdown instruction is received before stopping the write operation.

AccessPassword： Tag access password, 4 bytes, default all zeros.

SelectTagParam： Select the label parameter and specify the label to be written. The definition is the same as above.

WriteDataAry： Specify the content of the write tag, which can specify EPC, User, and reserved area at the same time, but do not specify them repeatedly, for example, you cannot specify two EPC areas at the same time.

- Received Message

The message has no further data to parse, and the Received Message is empty.

- Instruction Description

EPC length: 0-31 words, in units; Using characters as units;

User length: 0-127 characters, measured in words;

Reserved area length: 0-4 characters, measured in words;

Reserved data content: destroy password 2 words+access password 2 words

    - 6C Tag Lock (MgTagLock class)

Purpose: Label lock operation.

- Constructor function

C#：public MsgTagLock(LockTagParameter param) //

JAVA：public MsgTagLock(LockTagParameter param) //

Parameters:

param： The lock label parameter is defined as follows

public class LockTagParameter

{

public LockBank LockBank;

public LockType LockType;

public TagParameter SelectTagParam = null;

public byte[] AccessPassword = new byte[4];

}

public enum LockBank

{

All,           // 0, all data areas

TID,//1, TID area

EPC,//2, EPC area

User,          // 3. User data area

AccessPassword,// 4. Access password area

KillPassword   //5， Destroy password area

}

public enum LockType

{

Lock,          // 0, Locked

UnLock,        // 1. Unlock

PermanentLock, // 2. Permanently locked (reserved)

PermanentUnLock//3， Permanently unlocked (reserved)

}

AccessPassword： Tag access password, 4 bytes.

SelectTagParam： Select label parameters and specify the labels to be operated on. The definition is the same as above.

- Received Message

The message has no further data to parse, and the Received Message is empty.

- Instruction Description

The TID area of the EPC tag has been written with a TID code and set with an irreversible permanent lock at the factory, so unlocking the TID area will not be performed when unlocking the "all data area".

    - 6C Tag Destruction (MgTagKill class)

Purpose: Label destruction operation.

- Constructor function

C#：public MsgTagKill(KillTagParameter param)//

JAVA：public MsgTagKill(KillTagParameter param)//

Parameters:

param： The parameter for destroying tags is defined as follows

public class KillTagParameter

{

public byte[] KillPassword = new byte[4];//  Destroy password, 4 bytes

public TagParameter SelectTagParam = null;

}

KillPassword： Tag destruction password, 4 bytes.

SelectTagParam： Select label parameters and specify the labels to be operated on. The definition is the same as above.

- Received Message

The message has no further data to parse, and the Received Message is empty.

- Instruction Description

Destroying tags is an irreversible operation, please pay attention to data security.

    - 6C tag and PHASE value reading (using the sgTagReadWithPhase class)

Purpose: Universal reading of 6C labels and PHASE values.

- Constructor function

C #: public sgTagReadWithPhase()//Scan tag TID, unlimited time, unlimited times

Public sgTagReadWithPhase (ReadTagParameter param)//Scan tags

JAVA: public sgTagRead()//Scan tag TID, unlimited time, unlimited times

Public sgTagRead (ReadTagParameter param)//Scan tags

Parameters:

param： Read tag parameters, as mentioned in the previous section.

- Received Message

1) Label data

Purpose: To obtain tag data, which may include antenna number and RSSI data, depending on the configuration of Mg6CTagFieldConfig.

C#：public RxdTagData TagData {get;}

JAVA：public RxdTagData getTagData();

Return value: Label data. Same as the previous section.

Note: As mentioned in the previous section.

    - 6C Tag Inventory EPC Area and PHASE Value (MgTagInventoryWithPhase Class)

Purpose: Read EPC and PHARE values of 6C tags, and inventory of tags.

- Constructor function

C #: public sgTagInventoryWithPhase()//Scan tag EPC, unlimited time, unlimited times

Public sgTagInventoryWithPhase (InventoryTagParameter param)//Scan tag EPC

JAVA: public sgTagInventoryWithPhase()//Scan tag EPC, unlimited time, unlimited times

Public sgTagInventoryWithPhase (InventoryTagParameter param)//Scan tag EPC

Parameters:

param： Same section as above

- Received Message

1) Label data

Purpose: To obtain tag data, which only includes EPC. In addition, there may be antenna number, RSSI, and phase data, depending on the configuration of Mg6CTagFieldConfig.

C#：public RxdTagData TagData {get;}

JAVA：public RxdTagData getTagData();

Return value: Label data. Same as the previous section.

remarks:

Same as the previous section.

- Instruction Description

Same as the previous section.

- Common application functions and business scenario examples

This chapter provides further detailed explanations of RFID readers in common application businesses. The code part is developed using C # (vs2019) as an example.

  - Reader connection

There are two ways to connect a reader/writer: one is to directly specify the connection parameters, and the other is to obtain the connection parameters through a configuration file for connection. To connect multiple readers, simply create multiple instances of the Reader class. Attention: The name of the reader/writer cannot be duplicated.

Reader r;

private void Connect()

{

/*1. Initialize the reader/writer

*1.1 Supports 2 initialization methods

*2.1 Reader name cannot be duplicated

*

*/

//Method 1: It needs to be used in conjunction with the local "DeviceCpg. xml" file, with the same name as in the file

//r = new Reader("Reader1");

//Method 2: Specify the connection port type (recommended)

r = new Reader("Reader2", new TcpClientPort("192.168.1.100:9090"));

/*2. Connection

*

*/

var rspConnect = r.Connect();

if(!rspConnect.IsSucessed)

{

//Output connection error message

Trace. WriteLine ($"Connection failed! {rspConnect.ErrorInfo.ErrCode}:{rspConnect.ErrorInfo.ErrMsg}");

return;

}

/*3. Common event registration

*Pay attention to the issue of cross threading of business code in the event

*/

r.OnInventoryReceived += R_OnInventoryReceived; // Tag reading

r.OnTriggerInventoryState += R_OnTriggerInventoryState; // GPI status

r.OnBrokenNetwork += R_OnBrokenNetwork; // connection exception

/*4. Start reading

*

*/

//Reference tag reading example

/*5. Stop reading

*

*/

//Reference tag reading example

/*6. Event uninstallation

*

*/

r.OnInventoryReceived -= R_OnInventoryReceived; // Tag reading

r.OnTriggerInventoryState -= R_OnTriggerInventoryState; // GPI status

r.OnBrokenNetwork -= R_OnBrokenNetwork; // connection exception

/*7. Disconnect the connection

*

*/

r.Disconnect();

}

private void R_OnBrokenNetwork(string senderName, ErrInfo e)

{

//Information output and processing

//You can add reconnection control code here

}

private void R_OnTriggerInventoryState(string readerName, bool isInventory)

{

//Information output and processing

//Process the code that triggers the reading of application business

}

private void R_OnInventoryReceived(string readerName, NetAPI.Entities.RxdTagData tagData)

{

//Process and parse label data. Whether each field has a valid return value, refer to the settings before reading

}

  - Tag reading

Tag scanning is the process of quickly and continuously identifying data areas such as EPC, TID, User, etc. The instructions are detailed in section 3.27. Here we mainly introduce common scanning EPC and scanning TID. The sgTagInventory class is specifically used to scan tag EPCs, also known as inventory. The sgTagRead class is used for general tag reading functionality, and its parameterless constructor provides a shortcut to scan tag TIDs.

- The tag data is returned from the OnInventoryReceived listening event, which is on another thread outside the main thread. Please be aware when processing it. Tag scanning can only proceed after establishing a successful connection and loading listening events in section 4.1.

/*4. Start reading

*1.1 There are two types of reading methods

*/

//Type 1: Read only EPC, do not read TID USER

var rp = new ReadTagParameter();

rp.IsLoop = true;

rp.ReadCount = 0;

rp.TotalReadTime = 0;

rp.IsReturnEPC = true;

rp.IsReturnTID = false;

var msgInventory = new MsgTagInventory(rp);

if (r.Send(msgInventory))

{

//Message sent successfully, start reading

}

else

{

//Message sending failed, output log

Trace. WriteLine ($"Message sending failed! {msgInventory.ErrorInfo.ErrCode}:{msgInventory.ErrorInfo.ErrMsg}");

}

//Type 2: Read EPC TID、User

rp.IsReturnEPC = true;

rp.IsReturnTID = true;

rp.UserPtr = 0; // Starting address of User area

rp.UserLen = 12; // User area length. Different tags have different starting addresses and lengths, while also considering the situation when writing tags

var msgTagRead = new MsgTagRead(rp);

if (r.Send(msgTagRead))

{

//Message sent successfully, start reading

}

else

{

//Message sending failed, output log

Trace. WriteLine ($"Message sending failed! {msgTagRead.ErrorInfo.ErrCode}:{msgTagRead.ErrorInfo.ErrMsg}");

}

//Type 3: Automatic Reading Mode. After setting, the reader will immediately read and automatically read every time it is powered on

WorkMode readerWorkMode = WorkMode.AutoScanMode;

TagMB tagMB = TagMB.EPC | TagMB.TID | TagMB.User;// Set reading content

MsgReaderWorkModeConfig msg = new MsgReaderWorkModeConfig(readerWorkMode, tagMB);

if (r.Send(msgTagRead))

{

//Message sent successfully, start reading

}

/*5. Stop reading

*

*/

var msgStop = new MsgPowerOff();

if (r.Send(msgStop))

{

//Message sent successfully, stop reading

}

else

{

//Message sending failed, output log

Trace. WriteLine ($"Message sending failed! {msgStop.ErrorInfo.ErrCode}:{msgStop.ErrorInfo.ErrMsg}");

}

  - Other tag operations (write, lock, destroy)

The tag operation refers to the reading, writing, locking, and destroying of tags.

When writing a label, if the reader is scanning the label, please stop scanning first before performing the write operation.

General operating steps:

- Specify write operation label parameters (label selection)

- Specify the area and operation data for the operation label.

- Specify tag access password (destroy password), default to all 0s

- Send instructions for writing operations

- Based on the status returned by the write operation, the next step can be taken

    - Label writing

private void WriteTag()

{

//Label assignment

byte[] targetEPC = new byte[] { 0x12, 0x34, 0x56, 0x78 };// Tag EPC Content

TagParameter selectParam = new TagParameter();

selectParam.MemoryBank = MemoryBank.EPCMemory;

selectParam.Ptr = 0x00; // Select label parameters, starting address unit is bit

selectParam.TagData = targetEPC;

//Label writing

WriteTagParameter wp = new WriteTagParameter();

wp.SelectTagParam = selectParam;

wp.AccessPassword = new byte[4];// password

wp.WriteDataAry = new TagParameter[2];// Can write several sections simultaneously,

int p = 0;

//Write tags for EPC area

{

TagParameter tp = new TagParameter();

tp.MemoryBank = MemoryBank.EPCMemory;

//Write label parameters with the starting address unit in words

tp.Ptr = 0x02;

tp.TagData = new byte[] { 0x11, 0x22, 0x33, 0x44 };

wp.WriteDataAry[p] = tp;

p++;

}

//Write tags in the User section

{

TagParameter tp = new TagParameter();

tp.MemoryBank = MemoryBank.UserMemory;

tp.Ptr = 0x00;

tp.TagData = new byte[] { 0xaa, 0xbb, 0xcc, 0xdd };

wp.WriteDataAry[p] = tp;

}

MsgTagWrite msg = new MsgTagWrite(wp);

if (r.Send(msg))

{

String str="Write successful";

}

else

{

String str="Write failed: " + msg.ErrorInfo.ErrMsg;

}

}

    - Tag lock/unlock

private void TagLock()

{

//Label assignment

byte[] tagID = new byte[] { 0xE2, 0x80, 0x12, 0x34,0x56,0x78 };// Tag TID Content

TagParameter selectParam = new TagParameter();

selectParam.MemoryBank = MemoryBank.TIDMemory;

selectParam.TagData = tagID;

//Label operation

LockTagParameter lp = new LockTagParameter();

lp.SelectTagParam = selectParam;

lp.AccessPassword = new byte[] { 0x11, 0x22, 0x33, 0x44 }; // access password

lp.LockBank = LockBank.EPC; // Specify the area, see enumeration for details

lp.LockType = LockType.Lock;// lock/unlock

MsgTagLock msg = new MsgTagLock(lp);

if (r.Send(msg))

{

//Operation successful

}

}

    - Tag destruction (cautious operation)

private void TagKill()

{

//Tag selection

byte[] tagID = new byte[] { 0xE2, 0x80, 0x12, 0x34, 0x56, 0x78 };// Tag TID Content

TagParameter selectParam = new TagParameter();

selectParam.MemoryBank = MemoryBank.TIDMemory;

selectParam.TagData = tagID;

//Label operation

KillTagParameter lp = new KillTagParameter();

lp.SelectTagParam = selectParam;

lp.KillPassword = new byte[] { 0x11, 0x22, 0x33, 0x44 }; // Destroy password

MsgTagKill msg = new MsgTagKill(lp);

if (r.Send(msg))

{

//Operation successful

}

}

  - GPIO application

This chapter mainly introduces the common application businesses of GPIO. Please refer to the product manual for the possible number of interfaces and electrical characteristics of different products.

    - GPI

GPI： Universal input interface. GPI is usually used as a trigger signal (such as photoelectric sensors, radar) for access control, channel doors, item entry and exit statistics, and other application scenarios

/*

*This part of the code mainly uses GPI to trigger tag reading and determine the direction of entry and exit

*Requirement: Have 2 inputs (2 pairs of photoelectric sensors), and define 1 # ->2 # as the entry direction, and vice versa as the exit direction

*

*/

byte curPortNo = 0;// The currently triggered port

string direction = "in"; // Direction of entry and exit

private void R_OnGpiTriggerReceived(string readerName, MsgGpiQuery info)

{

byte portNo = info.ReceivedMessage.GpiLevel.PortNO;

//Determine the voltage level. Usually, only one type of change can be handled

if (GpioLevel.Low == info.ReceivedMessage.GpiLevel.Level)

{

return;

}

if (curPortNo ==0 || curPortNo == portNo)

{

//If the entry and exit process begins, or if the same port is triggered twice

curPortNo = portNo;

direction = portNo == 1 ?  "in" : "out";// Marking direction

this.StartRead(); // Refer to the "Tag Reading" chapter

}

else

{

//Triggering different ports twice in a row is a complete input/output

this.StopRead();

this.curPortNo = 0;// Reset state

}

}

private void R_OnInventoryReceived(string readerName, NetAPI.Entities.RxdTagData tagData)

{

//Analyze data and retrieve the data read during GPI triggering period

}

    - GPO

GPO： Universal output interface. Commonly used for controlling external devices such as engines, lights, alarms, etc

private void SetGPO()

{

var p = new GpioLevelParameter();

p.PortNO = 1;

p.Level = GpioLevel.High;

if(r.Send(new MsgGpoConfig(p)))

{

//Message sent successfully

}

}

  - Read mode description

Different reading modes are suitable for different application scenarios. The following is a detailed explanation and code example

Multi label mode: suitable for scenarios where more than 50 labels are read during the movement process, such as inbound and outbound storage;

Fast reading mode: suitable for application scenarios where tags need to be read quickly, such as conveyor belts, vehicle entry and exit, etc., where the number of tags to be read at once is less than 50;

Intensive reading mode: suitable for applications with multiple antennas and more than 200 tags. In a static environment, it is necessary to read all tags within a few seconds, such as RFID filing cabinets, RFID book cabinets, RFID consumables cabinets, etc;

Low power mode: suitable for scenarios where UHF RFID handheld devices or UHF RFID tablets are powered by batteries and require extended battery life;

public enum ScanMode

{

MutiTag,  // multi-label

SingleTag,// Fast reading

DenseMode,// Intensive reading

LowPower//Low power consumption

}

//Setting up

r.Send(new MsgScanModeConfig(ScanMode.MutiTag))

  - Other common configuration instructions

    - RSSI

RSSI： Received signal strength, usually negative (-110-0 dBm, 0 indicates not enabled), the smaller the value, the farther the distance. By setting appropriate RSSI values in the application, tags can be filtered, which to some extent reduces the rate of misreading and serial reading problems.

private void SetRSSI()

{

//Read setting range: 0-110. 0- Indicates not enabled

//When 0: parameter passes 0, when greater than 0: parameter passes complement 256-80

//Under specific readers and writers, RSSI has a specific algorithm. Please consult the manufacturer for details

byte rsii = 80;

MsgRssiThresholdConfig msg = new MsgRssiThresholdConfig(256-80);

if (r.Send(new MsgScanModeConfig(ScanMode.MutiTag)))

{

//Message sent successfully

}

}

    - Label lights up

Some tags support reading (activation) and their own indicator lights will light up to achieve guidance effect.

Please consult the label manufacturer for label parameters such as access password and reserved area address.

private void TaskLight()

{

/*

*Principle: Read the label reservation area and specify the location

*/

Task.Factory.StartNew(() =>

{

//Label parameters (specified)

TagParameter tp = new TagParameter();

tp.TagData = new byte[] { 0xE2, 0x80, 0x12, 0x34, 0x56, 0x78 }; // Awaiting Light Label TID

tp.MemoryBank = MemoryBank.TIDMemory;//

ReadTagParameter rtp = new ReadTagParameter();

rtp.SelectTagParam = tp;

rtp.AccessPassword = new byte[] { 0x00, 0x00, 0x00, 0x00 };// Change according to label

rtp.IsReturnEPC = false;

rtp.IsReturnTID = false;

rtp.IsReturnReserved = true;    // Read the reserved area (password area)

rtp.ResvPtr = 0x04; // Read the starting address. Modify according to the label

rtp.ResvLen = 0x01; // Read length

while (true)

{

var msg = new MsgTagRead(rtp);

r.Send(msg);

System.Threading.Thread.Sleep(1000);// If frequency control is required, it can be adjusted appropriately

}

});

}

    - Other common functions (tag deduplication, interval reading)

This chapter lists some commonly used functional APIs, with specific instructions referring to the corresponding API (Message object)

Tag deduplication: MsgFilteringTimeConfig(ushort time)

Interval reading: MsgIntervalTimeConfig(ushort readTime, ushort stopTime)

Appendix A: Example of Label Reading and Writing Process Controlled by Upper Computer Reader/Writer

The upper computer initiates a connection to the reader/writer through a specified port. After the connection is successful, the upper computer first connects to the reader/writer

The main purpose of sending a stop command is to switch the reader/writer to an idle state and respond normally to subsequent instructions

Order; 2. Confirm whether the current status of the reader is normal and available based on whether the response of the reader is successful.

Appendix B: Error Code Description

Error code description:

FF00 instruction does not return within the specified time

FF01 API Other Errors

FF02 reader/writer abnormal disconnection

FF03 configuration file error

FF04 reader/writer not connected or disconnected

FF05 message cannot be empty

F0 indicates instruction word error

F1 indicates instruction execution failure

F2 represents CRC error

F3 represents data frame format and frame parameter errors

F4 indicates that the storage area has exceeded the limit

F5 indicates that the storage area is locked

F6 indicates insufficient power

F7 indicates that the operation label is not supported

F8 means no tag detected or tag unresponsive

F9 indicates that the operation label has failed

0x12 antenna error

0x14 Write flash error

Appendix C: List of API Functions for Each Version

| message interface | C# | JAVA | Android | Android y2007 |
| --- | --- | --- | --- | --- |
| MsgReaderVersionQuery | ✔ | ✔ | ✔ | ✔ |
| MsgAntennaConfig | ✔ | ✔ | ✔ | ✔ |
| MsgAntennaStatusQuery | ✔ |  | ✔ |  |
| MsgRfidStatusQuery | ✔ | ✔ | ✔ | ✔ |
| MsgRs232BaudRateConfig | ✔ | ✔ | ✔ | ✔ |
| MsgRs485BaudRateConfig | ✔ | ✔ |  |  |
| MsgRs485AddressConfig | ✔ | ✔ |  |  |
| MsgGpioDefinitionQuery | ✔ | ✔ | ✔ |  |
| MsgGpiTriggerConfig | ✔ | ✔ | ✔ |  |
| MsgGpoConfig | ✔ | ✔ | ✔ |  |
| MsgGpiQuery | ✔ | ✔ | ✔ |  |
| MsgFilteringTimeConfig | ✔ | ✔ | ✔ | ✔ |
| MsgIntervalTimeConfig | ✔ | ✔ | ✔ | ✔ |
| MsgRssiThresholdConfig | ✔ | ✔ | ✔ | ✔ |
| MsgUhfBandConfig | ✔ | ✔ | ✔ | ✔ |
| MsgPowerConfig | ✔ | ✔ | ✔ | ✔ |
| MsgReadPowerConfig | ✔ |  | ✔ |  |
| MsgWritePowerConfig | ✔ |  | ✔ |  |
| MsgAirProtocolConfig | ✔ | ✔ | ✔ | ✔ |
| MsgFrequencyConfig | ✔ | ✔ | ✔ | ✔ |
| MsgResetToFactoryDefault | ✔ | ✔ | ✔ | ✔ |
| MsgIdleTimeConfig | ✔ | ✔ | ✔ | ✔ |
| MsgIpAddressConfig | ✔ | ✔ | ✔ |  |
| MsgTcpModeConfig | ✔ | ✔ |  |  |
| MsgMacConfig | ✔ | ✔ | ✔ |  |
| Msg6CTagFieldConfig | ✔ | ✔ | ✔ | ✔ |
| MsgTagInventory | ✔ | ✔ | ✔ | ✔ |
| MsgTagRead | ✔ | ✔ | ✔ | ✔ |
| MsgTagWrite | ✔ | ✔ | ✔ | ✔ |
| MsgTagLock | ✔ | ✔ | ✔ | ✔ |
| MsgTagKill | ✔ | ✔ | ✔ | ✔ |
| MsgTagInventoryWithPhase | ✔ |  | ✔ |  |
| MsgTagRead WithPhase | ✔ |  | ✔ |  |
| MsgTagFocusConfig | ✔ |  | ✔ |  |
| MsgFastIDConfig | ✔ |  | ✔ |  |
| MsgUtcConfig | ✔ | ✔ | ✔ | ✔ |
| MsgKeepAliveConfig | ✔ | ✔ | ✔ | ✔ |
| MsgAppCpuReset | ✔ |  | ✔ |  |
| MsgPortDwellTimeConfig | ✔ |  | ✔ |  |
| MsgReaderWorkModeConfig | ✔ |  | ✔ |  |
| MsgScanModeConfig | ✔ |  | ✔ |  |
| MsgPowerOff | ✔ | ✔ | ✔ | ✔ |
| MsgCpuVersionQuery | ✔ | ✔ |  |  |
| MsgTagSelect | ✔ | ✔ | ✔ | ✔ |
| MsgReaderCapabilityQuery | ✔ | ✔ | ✔ | ✔ |
| MsgWifiMacConfig | ✔ | ✔ |  |  |
| MsgWifiModeConfig | ✔ | ✔ | ✔ |  |
| MsgWifiIpAddressConfig | ✔ | ✔ | ✔ |  |
| MsgWifiHotspotConfig | ✔ | ✔ | ✔ |  |
| MsgWifiStatusConfig | ✔ | ✔ | ✔ |  |
| MsgQValueConfig | ✔ | ✔ | ✔ | ✔ |
| MsgSessionConfig | ✔ | ✔ | ✔ | ✔ |
| MsgReadTagCfgParamConfig | ✔ |  | ✔ |  |
| MsgReaderModuleSNQuery | ✔ |  | ✔ |  |
| MsgReaderSNQuery | ✔ |  | ✔ |  |
| MsgReaderBeepConfig | ✔ |  | ✔ |  |
| MsgSleepStateConfig | ✔ |  |  |  |
| MsgServerHostConfig | ✔ |  |  |  |
| MsgApnConfig | ✔ |  |  |  |
| Msg4GIMEIConfig | ✔ |  |  |  |
| MsgDeviceResetByUdp | ✔ |  |  |  |
| MsgDHCPModeConfig | ✔ |  |  |  |
| MsgDeviceResetByUdp | ✔ |  |  |  |
| MsgNetProtocolType | ✔ |  |  |  |
| MsgMQTTAuthorizeParam | ✔ |  |  |  |
| MsgMQTTBaseParam | ✔ |  |  |  |
| MsgWirelessCommunicationModeConfig | ✔ |  |  |  |

Appendix D: List of Partial Label Chip Manufacturers

| Manufacturer ID | Manufacturer name | Chip code | Chip name | sensitivity | EPC capacity | User capacity | Access password length | Destroy password length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | Impinj | 114 | Moza 4 | -19.5 | 256 | 480 | 32 | 32 |
| 01 | Impinj | 100 | Moza 4D | -19.5 | 128 | 32 | 32 | 32 |
| 01 | Impinj | 10C | Moza 4E | -19.5 | 496 | 128 | 32 | 32 |
| 01 | Impinj | 105 | Moza 4QT | -19.5 | 128 | 512 | 32 | 32 |
| 01 | Impinj | 130 | Moza 5 | -20 | 128 | 32 | 32 | 32 |
| 01 | Impinj | 160 | Moza R6 | -22.1 | 96 | 0 | 0 | 0 |
| 01 | Impinj | 170 | Moza R6-P | -22.1 | 96-128 | 32/64 | 32 | 0 |
| 01 | Impinj | 171 | MoZa R6-A | -22.1 | 96 | 0 | 32 | 32 |
| 01 | Impinj | 173 | Moza S6-C | -22.1 | 96 | 32 | 0 | 0 |
| 01 | Impinj | 191 | Moza M730 | -24 | 128 | 0 | 32 | 32 |
| 01 | Impinj | 190 | Moza M750 | -24 | 96 | 32 | 32 | 32 |
| 02 | Texas Instruments |  |  |  |  |  |  |  |
| 03 | Alien Technology | 412 | Higgs3 | -20 | 96-480 | 512 | 32 | 32 |
| 03 | Alien Technology | 414 | Higgs4 | -20.5 | 96/128 | 128 | 32 | 32 |
| 03 | Alien Technology | 811 | HiggsEC | -22.5 | 96/128 | 128 | 32 | 32 |
| 03 | Alien Technology | 821 | Higgs9 | -22.5 | 96-496 | 688 | 32 | 32 |
| 04 | Intelleflex |  |  |  |  |  |  |  |
| 05 | Atmel |  |  |  |  |  |  |  |
| 06 | NXP Semiconductors | 003 | G2XM | -15 | 240 | 512 | 32 | 32 |
| 06 | NXP Semiconductors | 004 | G2XL | -15 | 240 | 0 | 32 | 32 |
| 06 | NXP Semiconductors | 806 | G2iL | -18 | 128 | 0 | 32 | 32 |
| 06 | NXP Semiconductors | 807 | G2iL+ | -18 | 496 | 0 | 32 | 32 |
| 06 | NXP Semiconductors | 80A | G2iM | -17.5 | 256 | 512 | 32 | 32 |
| 06 | NXP Semiconductors | 80B | G2iM+ | -17.5 | 128-448 | 320-640 | 32 | 32 |
| 06 | NXP Semiconductors | 80D | UcodeFC | -18 | 160 | 3328 | 32 | 32 |
| 06 | NXP Semiconductors | 890 | Ucode7 | -21 | 128 | 0 | 32 | 32 |
| 06 | NXP Semiconductors | 811 | Ucode7ra | -21 | 128 | 32 | 32 | 0 |
| 06 | NXP Semiconductors | D12 | Ucode7xm | -19 | 448 | 1024 | 32 | 32 |
| 06 | NXP Semiconductors | D92 | Ucode7xm+ | -19 | 448 | 2048 | 32 | 32 |
| 06 | NXP Semiconductors | B12 | Ucode DXA | -19 | 224 | 1024 | 32 | 32 |
| 06 | NXP Semiconductors | 894 | Ucode 8 | -23 | 128 | 0 | 32 | 32 |
| 06 | NXP Semiconductors | 994 | Ucode 8m | -23 | 96 | 32 | 32 | 32 |
| 06 | NXP Semiconductors | 995 | Ucode 9 | -24 | 96 | 0 | 0 | 0 |
| 07 | ST Microelectronics |  |  |  |  |  |  |  |
| 08 | EP Microelectronics |  |  |  |  |  |  |  |
| 09 | Motorola |  |  |  |  |  |  |  |
| 0A | Sentech Snd Bhd |  |  |  |  |  |  |  |
| 0B | EM Microelectronics | 001 | 4324 | -8,- twenty-seven | 96 | 720 | 32 | 32 |
| 0B | EM Microelectronics | 002 | 4324 | -8,- twenty-seven | 96 | 720 | 32 | 32 |
| 0B | EM Microelectronics | XXX | 4325 | -8,- thirty | 320 | 3072 | 32 | 32 |
| 0B | EM Microelectronics | 0A0 | 4423 | -18 | 128/224 | 64/160 | 32 | 32 |
| 0B | EM Microelectronics | 010 | 4423 | -18 | 128/224 | 64/160 | 32 | 32 |
| 0B | EM Microelectronics | 080 | 4124 | -19 | 96 | 0 | 32 | 32 |
| 0B | EM Microelectronics | 0C0 | 4126 | -19 | 208 | 0 | 0 | 0 |
| 0C | Renesas Technology Corp. |  |  |  |  |  |  |  |
| 0D | Mstar |  |  |  |  |  |  |  |
| 0E | Tyco International |  |  |  |  |  |  |  |
| 0F | Quanray Electronics |  |  |  |  |  |  |  |
| 10 | Fujitsu |  |  |  |  |  |  |  |
| 11 | LSIS |  |  |  |  |  |  |  |
| 12 | CAEN RFID srl |  |  |  |  |  |  |  |
| 13 | Productivity Engineering GmbH |  |  |  |  |  |  |  |
| 14 | Federal Electric Corp. |  |  |  |  |  |  |  |
| 15 | ON Semiconductor |  |  |  |  |  |  |  |
| 16 | Ramtron |  |  |  |  |  |  |  |
| 17 | Tego |  |  |  |  |  |  |  |
| 18 | Ceitec S.A. |  |  |  |  |  |  |  |
| 19 | CPA Wernher von Braun |  |  |  |  |  |  |  |
| 1A | TransCore |  |  |  |  |  |  |  |
| 1B | Nationz |  |  |  |  |  |  |  |
| 1C | Invengo |  |  |  |  |  |  |  |
| 1D | Kiloway | 011 | KX2005X-B |  |  |  |  |  |
| 1D | Kiloway | 012 | KX2005X-S |  |  |  |  |  |
| 1D | Kiloway | 013 | KX2005X-544/512 |  |  |  |  |  |
| 1D | Kiloway | 014 | KX2005X-240 |  |  |  |  |  |
| 1D | Kiloway | 015 | KX2005X-BL |  |  |  |  |  |
| 1D | Kiloway | 016 | KX2005X-BT |  |  |  |  |  |
| 1D | Kiloway | 017 | KX2005X-BR |  |  |  |  |  |
| 1D | Kiloway | 021 | KX2005XG-B |  |  |  |  |  |
| 1D | Kiloway | 022 | KX2005XG-S |  |  |  |  |  |
| 1D | Kiloway | 023 | KX2005XG-544 |  |  |  |  |  |
| 1D | Kiloway | 030 | KE2006 |  |  |  |  |  |
| 1D | Kiloway | 061 | ONLY1 |  |  |  |  |  |
| 1D | Kiloway | 062 | ONLY2 |  |  |  |  |  |
| 1E | Longjing Microelectronics Co. Ltd. |  |  |  |  |  |  |  |
| 1F | Chipus Microelectronics |  |  |  |  |  |  |  |
| 20 | ORIDAO |  |  |  |  |  |  |  |
| 21 | Maintag |  |  |  |  |  |  |  |
| 22 | Yangzhou Daoyuan Microelectronics Co. Ltd |  |  |  |  |  |  |  |
| 23 | Gate Elektronik |  |  |  |  |  |  |  |
| 24 | RFMicron, Inc. |  |  |  |  |  |  |  |
| 25 | RST-Invent LLC |  |  |  |  |  |  |  |
| 26 | Crystone Technology |  |  |  |  |  |  |  |
| 27 | Shanghai Fudan Microelectronics Group |  |  |  |  |  |  |  |
| 28 | Farsens |  |  |  |  |  |  |  |
| 29 | Giesecke & Devrient GmbH |  |  |  |  |  |  |  |
| 2A | AWID |  |  |  |  |  |  |  |
| 2B | Unitec Semicondutores S/A |  |  |  |  |  |  |  |
| 2C | Q-Free ASA |  |  |  |  |  |  |  |
| 2D | Valid S.A. |  |  |  |  |  |  |  |
| 2E | Fraunhofer IPMS |  |  |  |  |  |  |  |
| 2F | ams AG |  |  |  |  |  |  |  |
| 30 | Angstrem JSC |  |  |  |  |  |  |  |
| 31 | Honeywell |  |  |  |  |  |  |  |
| 32 | Huada Semiconductor Co. Ltd (HDSC) |  |  |  |  |  |  |  |
| 33 | Lapis Semiconductor Co., Ltd. |  |  |  |  |  |  |  |
| 34 | PJSC Mikron |  |  |  |  |  |  |  |
| 35 | Hangzhou Landa Microelectronics Co., Ltd. |  |  |  |  |  |  |  |
| 36 | Nanjing NARI Micro-Electronic Technology |  |  |  |  |  |  |  |
| 37 | Southwest Integrated Circuit Design Co., Ltd. |  |  |  |  |  |  |  |
| 38 | Silictec |  |  |  |  |  |  |  |
| 39 | Nation RFID |  |  |  |  |  |  |  |
| 3A | Asygn |  |  |  |  |  |  |  |
| 3B | Suzhou HCTech Technology Co., Ltd. |  |  |  |  |  |  |  |
| 3C | AXEM Technology |  |  |  |  |  |  |  |
| 3D | Guangzhou Syschip Technology Co., Ltd |  |  |  |  |  |  |  |
| 3E | MaxWave Microelectronics Ltd. |  |  |  |  |  |  |  |
| 3F | IDRO Co., Ltd |  |  |  |  |  |  |  |